# Closed-Loop Ball Demos — Operator Usage Guide

How to USE the system. The companion design spec at closed_loop_ball_demos.md covers WHY each piece exists.

Read this when you’re about to drive the robot. Read the spec when you want to know why a particular choice was made.

This guide assumes the v9 spec / v8.1 GUI scaffold is live on the Pi. The cold-start sequence below mirrors the existing stable_bot_startup.txt reference card; everything else is new with the vision pipeline.

## 0. Pre-flight before pushing v8.1 GUI to the Pi

Run these once after pulling the latest commit, before walking up to the robot:

- ☐ git pull on the Pi to grab the new GUI + nodes:

- ssh sorak@stablebot.local
cd ~/<your-workspace>/src/stable_bot_repo && git pull

- ☐ Build the new package (intrinsics + extrinsics calibrators, ArUco helpers, calibration node, bag recorder):

- cd ~/<your-workspace>
source /opt/ros/kilted/setup.bash
colcon build --packages-select stewart_vision
source install/local_setup.bash

- ☐ Restart the GUI service so the new index.html is served:

- sudo systemctl restart stable_bot_gui.service

- ☐ Open http://stablebot.local:8080/ from your laptop. Confirm:

  - Vision Debug + Calibration panels appear at the top of the LEFT column.

  - Demo 1 / 2 / 3 panels appear at the top of the MIDDLE column.

  - Existing controls (Status / Arming / Encoders / Jog / Pose / Homing / ODrive errors / etc.) still work unchanged.

  - The 3D Viewer panel is collapsed at the bottom of the middle column (placeholder; v9 implementation).

- ☐ If the OAK isn’t plugged in yet, the page will still load — oak_driver_node will log a fatal but the existing control stack is unaffected. Plug in the OAK and sudo systemctl restart stable_bot.service to bring vision online.

- ☐ Run the Stage-C calibration flow (§2). Confirm oak_extrinsics.yaml gets written.

- ☐ Pick the ball type in the GUI header dropdown (“Ball: foam” vs “Ball: ping-pong”) to match the physical ball you’re about to use. Persists across reloads. Affects PID feedforward (α=5/7 solid vs 3/5 hollow) and is captured in every bag.

- ☐ Optional: run the unit tests on your laptop before pushing:

- cd ~/stable_bot_repo/stewart_vision
python3 -m pytest test/ -v

- 35 should pass; up to 3 cv2-aruco-dependent tests will skip if opencv-contrib isn’t installed in the env you’re testing from.

Once all checkboxes pass, you’re at step 1 of the iteration ladder in spec §16.

## 1. Cold start (≈ 5 minutes from power-off to first demo)

### 1.1 Pre-flight check

- ☐ Pixel 9 hotspot is on; laptop is connected to it.

- ☐ Stable-Bot sits on a stable, level workbench.

- ☐ OAK-D Pro AF is on its camera arm, pointed at the platform, cable plugged into the Pi’s blue USB-A 3.0 port (NOT USB-C — that’s power-only on the Pi 5).

- ☐ Both MTi-630 IMUs and the USB-CAN adapter are plugged in.

- ☐ No ball on the platform yet.

### 1.2 Power on

- Plug Stable-Bot’s power into the wall.

- Wait ~45 seconds for the Pi to boot. Two systemd services auto-launch:

  - stable_bot.service — ROS 2 nodes (control, IMU, vision)

  - stable_bot_gui.service — gui_server.py + rosbridge_websocket

### 1.3 Connect the laptop browser

- Open http://stablebot.local:8080/ in your browser. (Hotspot mDNS sometimes flakes — if mDNS doesn’t resolve, look up the IP in the Pixel hotspot’s “Connected devices” page next to stablebot and use http://<pi-ip>:8080/.)

- The page should load. The header connection dot turns green within ~5 seconds.

- The header WS URL field defaults to ws://stablebot.local:9090. It is sticky (last successful URL is remembered). If you needed to use a hotspot IP for HTTP, also override the WS URL to match.

- If you see “no /status received” for > 10 seconds, click the red RESET button in the Diagnostics panel. The whole stack restarts; wait 30 seconds and try again.

### 1.4 Arm + level the platform

- Arm — click the green Arm button (left column, Arming panel). All six legs go to CLOSED_LOOP_CONTROL.

- Go to rest — settles the top plate to neutral pose.

- Level ON — IMU-based level loop holds the plate flat (within ~0.2°).

If anything misbehaves (Errors panel turns red, motors won’t arm), use Diagnostics → Hard reset stack. That kills and re-launches everything from a clean slate.

## 2. Camera calibration (Stage C only — once per camera arm move)

The OAK ships factory-calibrated for intrinsics and stereo extrinsics, and the GUI uses those by default. The only thing you re-run is Stage C — the camera-to-platform pose — and only when the camera arm physically moves.

If the camera hasn’t moved since your last calibration, skip this section.

### 2.1 Pre-flight

- ☐ Platform is leveled (Level ON; confirm IMU readout shows roll and pitch within ±0.1°).

- ☐ ArUco ring sticker is intact, all 8 markers visible from the current camera angle.

- ☐ Lighting is even on the ring (no glare from the IR projector).

### 2.2 Capture + solve in the GUI

- In the Vision Debug panel (left column, top), expand the Calibration (Stage C) sub-panel.

- Confirm the live left-eye feed shows the ArUco markers with green corner overlays drawn on each. The “markers visible” count should read 8 / 8 (or at minimum 3 / 8 to proceed).

- Click Capture frame. A toast reports e.g. “frame 1 accepted — 8/8 markers, reproj 0.74 px”. If reprojection > 2 px or markers < 3, the frame is rejected; adjust angle/lighting and retry.

- (Optional) capture 2-3 more frames at slightly varied platform states for better averaging — not necessary on a stable leveled platform but doesn’t hurt.

- Click Solve. The system runs cv2.aruco.estimatePoseBoard over the captured frames, averages, and writes oak_extrinsics.yaml into the package’s share directory.

- Confirm the toast: “oak_extrinsics.yaml written — 1 frame, mean reproj 0.74 px. Buffer cleared.”

You’re done. The platform_pose node picks up the new extrinsics on its next pose update; no restart required.

### 2.3 If something fails

| Toast / error | Cause | Fix |
| --- | --- | --- |
| “no intrinsics loaded” | First-time setup — factory cal hasn’t been read yet | SSH to the Pi: python3 ~/<repo>/stewart_vision/scripts/calibrate_oak.py --stage factory. Reads OAK EEPROM, writes oak_intrinsics.yaml in < 2 s. |
| “only N/8 markers visible” | Ring partially out of frame, glare, or lighting | Improve lighting, rotate camera arm, or rotate the platform manually so all 8 markers are in view. |
| “reproj exceeds 2 px gate” | Platform isn’t actually leveled, or marker_layout.yaml doesn’t match the printed ring | Re-level (Level ON; check IMU). Verify the ring radius matches marker_layout.yaml (ring_radius_mm: 120.0). |
| “estimatePoseBoard returned 0” | Detection geometry is degenerate (rare) | Reset, capture a different angle. |

### 2.4 Advanced (CLI on the Pi only)

Stages A and B (custom intrinsics + stereo recalibration) override the OAK’s factory tuning and are deliberately not in the GUI — a misclick could replace a working factory cal with a half-baked custom one.

If you genuinely need them:

ssh sorak@stablebot.local
cd ~/<your-workspace>/src/stewart_vision/scripts
python3 calibrate_oak.py --stage A    # ~30 chessboard captures
python3 calibrate_oak.py --stage B    # stereo extrinsics

These are rare maintenance operations. The factory calibration is within OAK’s published spec (≤ 0.3 px reprojection); your custom one might be slightly tighter, but the demo is not bottlenecked by this.

## 3. The Vision Debug panel

Top of the left column. Collapsible — collapse it during demos to keep the screen clean; expand it whenever something looks wrong.

### 3.1 Live camera feed

- 15 fps JPEG-compressed RGB stream.

- Frames drop silently when the hotspot link is congested. The control loop is unaffected — it runs at full rate on the Pi regardless of what the laptop sees.

- FPS indicator shows the actual delivered rate. If it drops below ~5 fps, the laptop link is your bottleneck — close any bandwidth-heavy apps.

Toggles:

| Toggle | Effect |
| --- | --- |
| Show stereo | Replaces the RGB feed with side-by-side rectified left + right pair. Useful for verifying rectification (epipolar lines should be horizontal). |
| Show disparity | Replaces the feed with the OAK’s color-mapped SGBM disparity map. Useful for verifying depth coverage of the platform. |

### 3.2 Telemetry strip

| Indicator | What it means | Healthy range |
| --- | --- | --- |
| ArUco markers visible | Markers detected by platform_pose_node (n / 8) | 6+ during demos; 8 / 8 ideal |
| KF NIS | Normalized Innovation Squared from ball_kf_node | 0.5 – 5 most of the time |
| End-to-end latency | OAK exposure → /ball_state on the laptop | < 40 ms |
| OAK-vs-mono | mm error between OAK truth and classical mono ray-plane | green < 5 mm |
| OAK-vs-stereo | mm error between OAK truth and classical stereo triangulation | green < 5 mm |
| mono-vs-stereo | mm disagreement between the two classical methods | green < 5 mm |

Color coding for the three method-comparison KPIs:

- green — |error| < 5 mm — methods agree.

- yellow — 5 mm ≤ |error| < 15 mm — minor disagreement; keep watching but no action needed.

- red — |error| ≥ 15 mm — significant disagreement; one method has an issue. The CONTROL loop is unaffected (it only uses OAK truth) but you should investigate after the run.

## 4. Demo 1 — Orbit at fixed radius

The ball orbits the platform center at a user-set radius and period.

### 4.1 Pre-flight

- ☐ Calibration is fresh (Stage C done since last camera move).

- ☐ Platform is armed and Level ON.

- ☐ Vision Debug shows ≥ 6 ArUco markers visible.

- ☐ Foam ball placed on the platform anywhere outside the red dead-zone disk.

### 4.2 Run the demo

- Open the Demo 1 — Orbit panel (middle column).

- Set:

  - Radius (35 to ~195 mm)

  - Period (1 to 10 s — one full orbit)

  - Direction (CW / CCW)

- Click Start. The platform first drives the ball to the orbit radius (acquisition phase, ~2-4 s), then engages the orbit reference. The mini-SVG disk shows the reference circle and the measured ball trail.

- Mid-orbit you can press the UNO-reverse button (yellow, two curved arrows) to flip direction. The ball decelerates through zero and accelerates the other way — a good visual stress test of the controller.

- Click Stop when done. The platform drops to LEVEL_HOLD.

### 4.3 What to expect

- Tracking error should be ≤ 5 mm RMS once warmed up.

- Latency in the Vision Debug strip should stay < 40 ms.

- If the ball wanders, check latency and KF NIS first.

## 5. Demo 2 — Click-to-Goto

The ball stays put until you click somewhere on the SVG disk; then the controller drives the ball to that point and holds.

### 5.1 Pre-flight

Same as Demo 1.

### 5.2 Run the demo

- Open the Demo 2 — Click-to-Goto panel.

- Click Start to arm goto mode.

- Click anywhere on the SVG disk that’s:

  - Inside the platform circle (200 mm radius).

  - Outside the red dead-zone (~30 mm-radius disk at center).

- The ball drives to that point and holds. Current target is a hollow blue ring; ball is the filled blue dot.

- Click another spot — the ball cancels and re-targets immediately. No queueing.

- Cancel target clears the reference (ball stays put, LEVEL_HOLD).

- Stop disarms Demo 2 mode.

### 5.3 Toggles

| Control | Effect |
| --- | --- |
| Predicted path (default off) | Dashed line from current ball position to target. Useful for debugging the PID’s approach trajectory. |
| Z-height slider | Pure heave on the top plate (no tilt). 0 to 80 mm above neutral. Adjustable while a goto is in progress. |

### 5.4 Failure handling

If the ball is bumped off the platform mid-demo, the off-platform watchdog fires (see §7).

## 6. Demo 3 — Path drawing (stretch goal)

You draw a freehand path on the SVG disk; the ball follows it.

### 6.1 Drawing a path

- Open the Demo 3 — Path drawing panel.

- Click Draw mode to enable freehand drawing on the SVG.

- Click and drag across the disk to trace a path. Sampling rate is ~30 ms per point.

- As you draw, valid segments render in green; segments that cross the dead-zone or go off-platform render in red.

- Release the mouse to finish. If any red segments exist, Start stays disabled — re-draw to clean up.

### 6.2 Running the path

- Set:

  - Loop — checkbox; closes the path with a smooth join and repeats indefinitely until Stop.

  - Traversal speed — slider, default 80 mm/s.

- Click Start. The ball drives to the path’s start (using Demo 2’s goto law), then traverses the path at the set speed.

- UNO-reverse mid-traversal flips the direction at the ball’s current path position. Trail-erase state resets on reverse.

- Stop drops to LEVEL_HOLD. The drawn path is preserved — pressing Start again resumes from the start.

### 6.3 Display modes

| Mode | Effect |
| --- | --- |
| Trail-erase (default) | Segment behind the ball greys out as the ball passes. Best “you are here” cue. |
| Static green | Whole polyline stays bright green throughout. Toggle in the panel. |
| Predicted direction | Faint arrows along the polyline indicate traversal direction. |

### 6.4 Save / load / export

- Save to a named slot in browser localStorage. Survives page reloads and browser restarts.

- Load picks a saved slot; the path appears on the disk ready to Start.

- Export downloads the current path as .json (schema_version 1). Check it into git, share via email, etc.

- Import uploads a .json and loads it as the current path.

## 7. Safety system

### 7.1 Dead-zone (the red disk at the center)

Represents the ball’s no-go zone. The IMU bolt protrudes through the platform’s geometric center; the ball must never touch it. The disk’s radius is 30 mm (10 mm clearance + 20 mm ball radius).

- Demo 2 clicks inside the dead-zone flash and are ignored.

- Demo 3 paths that cross the dead-zone are rejected with a re-draw prompt.

- Demo 1 orbit radius is silently clamped to ≥ 35 mm.

- Soft repulsion in the controller adds an outward tilt term if the measured ball strays into the dead-zone + margin — this kicks in BEFORE the ball can touch the bolt, even if the reference is somehow trying to drive it inward.

### 7.2 Off-platform watchdog

If the measured ball position has radius > 200 mm from center (off the platform) OR both detectors lose it for > 500 ms:

- Platform drops to LEVEL_HOLD immediately (silent).

- GUI body background strobes red ~3 Hz for 3 seconds.

- Background settles to a dim red tint until you re-arm.

- A toast appears: “Ball lost — platform leveling. Replace ball on platform and press Start to re-arm.”

To re-arm:

- Place the ball back on the platform within the 180 mm hysteresis radius.

- Wait ~1 second for the detection to confirm (the strobe dims).

- Press Start in whichever demo panel was active.

Re-arm is manual by design — automatic resume while your hand is still on the platform would be surprising.

### 7.3 Emergency stop

The big red E-STOP button at the top of the GUI calls /e_stop. All six leg drives go to IDLE immediately. Use this if anything looks wrong — runaway motion, smoke, unexpected geometry, etc.

E-STOP requires manual recovery: arm again from scratch.

## 8. Shutdown

Always shut the Pi down cleanly before pulling power, or you’ll corrupt the SD card.

ssh sorak@stablebot.local
sudo shutdown -h now

Wait for the Pi’s green activity LED to stop blinking (~10 s), then unplug power.

## 9. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Page loads but “no /status received” | Stack didn’t fully start | Click Diagnostics → Hard reset stack. Wait 30 s. |
| Connection dot stays red after Connect | WS URL wrong, mDNS blocked, or rosbridge down | Use IP-based URL; if still red, SSH and systemctl status stable_bot_gui. |
| OAK not detected on the Pi | USB cable / port issue | SSH and lsusb should show 03e7:f63b (Movidius MyriadX). Reseat cable into the blue USB-A 3.0 port. |
| ArUco markers detected but Stage C won’t solve | Platform not actually leveled OR marker_layout.yaml mismatch | Re-level (Level ON; IMU within 0.1°). Verify marker_layout.yaml ring radius matches your printed ring. |
| Demo 1 orbit oscillates / wobbles | Hotspot link congested or PID tuning | Check Vision Debug latency. If > 50 ms, drop period to 5 s. If link is fine, may need to retune PID gains. |
| GUI clicks don’t respond | Connection state stale | Click Connect to force reconnect. |
| Strobing red won’t clear | Re-arm condition not met | Place ball within 180 mm radius, wait 1 s, press Start. |
| FPS readout shows 0 / very low | Hotspot bandwidth | Restart hotspot. Or, if just debugging on a desk near the Pi, switch the laptop to Pi’s WiFi if available. |
| Method-comparison KPIs all red | Bad calibration extrinsics | Re-run Stage C calibration. |
| KF NIS consistently > 10 | Q or R noise mismatched | Check Vision Debug latency — high latency often masquerades as bad noise tuning. If latency is fine, the ball state KF needs Q/R retuning (advanced; see spec §8). |

## 10. Cheat sheet (one-page reference)

Cold start:
    Hotspot on → Pi power → wait 45 s → http://stablebot.local:8080/
    Connect → Arm → Go to rest → Level ON

Calibration (only after camera arm moves):
    Vision Debug → Calibration sub-panel
    8/8 markers visible → Capture frame → Solve

Demo 1 (Orbit):
    Set radius / period / direction → Start
    UNO-reverse mid-orbit if you want to flip
    Stop

Demo 2 (Click-to-Goto):
    Start → click any safe point → ball goes there → click again to retarget
    Stop / Cancel target

Demo 3 (Path):
    Draw mode → trace a path → Loop? → set speed → Start → Stop

If ball falls off → page strobes red → put ball back → Start to re-arm
Anything looks wrong → E-STOP → diagnose → re-arm from scratch