# Closed-Loop Ball Demos — Vision-Based Control Plan

Project: Stable-Bot (6-DOF Stewart platform, ROS 2 Kilted) Scope: Replace the open-loop rolling-ball demo with a vision-driven closed loop. Three demos: orbit (Demo 1), click-to-position (Demo 2), and path-drawing follow (Demo 3, stretch goal). Authors: Luke McDevitt + Claude (Opus 4.7). Last revised: 2026-04-26 (v7 — GUI calibration button is Stage C only; Stages A/B are CLI-only to prevent operators accidentally overriding factory tuning; clarified that the existing web/index.html is already three-column).

This plan supersedes the relevant sections of stewart_bringup/docs/NEXT_STEPS.md. It pulls in lecture material from EEL 4930/5934 (Lectures 6 — Robot Perception, 8 — Filtering & PID).

## 1. Status and goals

- Open-loop rolling-ball demo previously failed the professor’s review.

- New requirement: real closed loop using vision.

- Extra-credit: drive the ball to any operator-specified point on the platform.

- Hardware has been purchased; bring-up is the work that remains.

The two demos defined below are the minimum viable deliverables. Both use the same vision pipeline; only the reference generator differs.

## 2. Hardware summary

| Component | Detail |
| --- | --- |
| Camera | Luxonis OAK-D-PRO-AF — stereo + 12 MP color + IR dot projector + Movidius Myriad X VPU. Onboard inference frees the Pi. |
| Camera mount | 5 ft aluminum rod fixed to the bottom platform (clear of actuators and the moving top plate), with an articulated camera arm + ball-head mount so the camera angle and height can be adjusted between trials. |
| Ball | 40 mm foam ball, mass-matched to a ping-pong ball. Solid foam → moment of inertia I = (2/5)·m·R² instead of the hollow-sphere (2/3)·m·R². See §9 for the control implication. |
| Platform | 400 mm-diameter top plate; IMU bolt protrudes through the geometric center. |
| ArUco ring | Pre-printed via generate_aruco_ring.py (DICT_4X4_50, 8 markers, 50 mm side, ring radius 120 mm). marker_layout.yaml accompanies the print. |
| IMU | Movella MTi-630 AHRS, mounted parallel to the top plate; platform_level.yaml references roll/pitch zero to the IMU’s factory-calibrated frame (see the MTi reference-frame discussion in chat history dated 2026-04-25). |
| Compute | Raspberry Pi 5, 16 GB RAM, ROS 2 Kilted. Phase 8 migration is complete (2026-04-26). Two systemd services auto-start the stack on power-up: stable_bot.service (ROS 2 nodes) + stable_bot_gui.service (gui_server.py + rosbridge). Pi 5 USB-C is power-only — OAK plugs into a blue USB-A 3.0 port (dedicated PCIe-fed bandwidth, no Pi 4 shared-controller bottleneck). |
| Network | Laptop ↔ Pi over the user’s Pixel 9 phone hotspot (same LAN). Hostname stablebot; current IP is hotspot-DHCP-assigned (look up in the hotspot’s “Connected devices”). GUI is at http://<pi-ip>:8080/; rosbridge websocket at ws://<pi-ip>:9090/. SSH user: sorak. |
| Bring-up laptop | Used only to host the browser. WSL2 Ubuntu 24.04 retained for development + GitHub flow. |

The articulated mount is a deliberate choice — the camera angle WILL be re-tuned during bring-up. Every camera move invalidates the extrinsics; intrinsics survive. The calibration workflow in §6 is designed around this.

## 3. The two demos

### Demo 1 — Orbit at fixed radius (closed loop)

The ball orbits the platform center at a user-specified radius and period. The demo starts from wherever the ball happens to be sitting: the controller first drives it to the orbit radius (re-using Demo-2’s goto law), then engages the orbit reference. No operator interaction during the run beyond Start / Stop.

- Acquisition phase: Demo-2 goto law to (R, 0).

- Orbit phase reference: (x*(t), y*(t)) = (R·cos(2πt/T + φ), R·sin(2πt/T + φ)).

- R is bounded below by the dead-zone (§10) and above by platform_radius − ball_radius − margin.

- T (period), direction, and R are GUI-selectable.

- Termination: Stop button, ball lost > 500 ms, or platform fault.

### Demo 2 — Click-to-position (extra credit, primary deliverable)

The ball sits stationary at its current location until the operator clicks a different point on the GUI’s graphical disk view; the controller then drives the ball to that point and holds.

GUI for Demo 2 is a graphical (SVG) representation of the platform, not a camera feed. It displays:

- The platform circle with the ArUco ring drawn at r = 120 mm.

- The dead-zone disk (red, see §10) at the center.

- The current ball position (filled dot) updated from real vision data.

- The current ball reference (target marker, hollow ring) — clicking inside the platform-but-outside-dead-zone publishes a new reference.

- A z-height slider (commands pure heave on the Stewart top plate; no tilt change; range 0 to 80 mm above neutral pose).

- Predicted-path overlay (toggled by checkbox, default off): a dashed line from current ball position to the target.

- Start / Stop buttons (mandatory safety). A second click while in flight cancels the current target and retargets immediately.

The live stereo camera feed remains available in a separate “Vision Debug” panel (§11) but is not the primary view for Demo 2.

### Demo 3 — Path drawing & follow (stretch goal)

Operator drags the cursor across the SVG disk to draw a freehand path; the ball is driven first to the path’s start (Demo-2 goto law) and then follows the drawn polyline at a configurable traversal speed. Optional “loop” checkbox closes the path with a smooth join from end to start and repeats indefinitely until Stop.

- Path captured as a sampled polyline (one (x_mm, y_mm) point every ~30 ms during draw), then smoothed (e.g. 5-point moving average) and resampled at constant arc-length spacing.

- Reference generator parameterizes the smoothed polyline by time at the operator-set traversal speed (default 80 mm/s, slider).

- Loop mode: closes path with a cubic-spline join end→start, modulo traversal.

- Pre-flight rejection: if any path sample falls inside the dead-zone or off the platform edge, the path is rejected with a UI flash — user must redraw. (Auto-clipping was considered but introduces surprise: a redrawn path is more honest than a silently-mutated one.)

- Start / Stop buttons (mandatory). Stop drops to LEVEL_HOLD.

- Predicted-path checkbox shows the upcoming traversal direction as a faint arrow chain along the polyline.

This is gated on Demos 1 and 2 working cleanly. Specification details that need confirmation are in §15 Q11–Q14.

### Why this order

Demo 1 stresses the loop’s ability to track time-varying setpoints (differentiator, feed-forward). Demo 2 stresses settling and the safety machinery for arbitrary targets. Demo 3 combines both. All three share everything below the reference generator; debugging is additive.

## 4. Coordinate frames

We will publish the ball position in both frames; subscribers pick what they need.

| Frame | Symbol | Origin | Axes | When to use |
| --- | --- | --- | --- | --- |
| World | W | Bottom platform center, gravity-aligned | +z up (gravity-anti-parallel), +x toward the operator | Camera extrinsics, GUI graphical view, planning |
| Platform | P | Top plate center, rotates+translates with the plate | +z normal to plate, +x along the IMU’s reported +x | Controller error computation, ball dynamics |
| Camera | C | OAK-D left optical center | OpenCV convention (+z forward, +x right, +y down) | Vision math only |
| ArUco board | B | Center of the marker ring on the plate (= platform origin by design) | Same as P | Pose link from C to P |

Transform chain: C → B (= P) → W via the ArUco-board pose every frame, plus a one-shot C → W from a known-position calibration marker laid on the leveled platform.

## 5. Vision stack architecture (new ROS 2 package)

stewart_vision is a separate package from stewart_bringup. This keeps the controller portable for users who want to swap in a different vision stack (e.g. external mocap, RealSense, mono webcam). Topics are the contract; node implementations are interchangeable.

stewart_vision/
  stewart_vision/
    oak_driver_node.py       # DepthAI pipeline, raw + on-VPU detections
    rectifier_node.py        # publishes rectified pairs on demand (GUI)
    platform_pose_node.py    # ArUco board → /platform_pose @ 30 Hz
    ball_localizer_node.py   # mono-projection AND triangulation paths
    ball_kf_node.py          # constant-vel KF (or EKF) → /ball_xy @ 100 Hz
    ref_generator_node.py    # /ball_ref from orbit OR click-to-goto
  scripts/
    calibrate_oak.py         # one-shot intrinsics + stereo + extrinsics
    aruco_pose_demo.py       # standalone ArUco-board sanity tool
    sfm_demo.py              # SIFT→F→E→R,t pipeline (Lecture 6)
    epipolar_overlay.py      # rectification verification, drawn on pairs
  config/
    marker_layout.yaml       # produced by generate_aruco_ring.py
    oak_intrinsics.yaml      # produced by calibrate_oak.py
    oak_extrinsics.yaml      # produced by calibrate_oak.py (re-run on every camera move)
    ball_safety.yaml         # dead-zone + margins + loss timeout
  launch/
    stewart_vision_launch.py
  docs/
    closed_loop_ball_demos.md   # this file (final home)

stewart_control_node (in stewart_bringup) gains a BALL_TRACK mode that subscribes to /ball_state + /ball_ref and runs the controller described in §9. No other changes to stewart_bringup are required.

## 6. Camera calibration workflow

Calibration runs on the Pi (where the OAK is plugged in). The user-facing flow is split by audience (locked v7, 2026-04-26):

- GUI button — Stage C only. The laptop’s GUI exposes capture / solve / reset for the camera-to-world ArUco-ring step only. This is the operation that matters between demo runs (every camera arm move re-runs it) and is the only one a non-developer operator ever needs.

- CLI only — --stage factory, --stage A, --stage B. Reading factory cal from EEPROM and any custom chessboard recalibration happen only via SSH + python3 calibrate_oak.py on the Pi. Reasoning: A and B override the OAK’s factory-tuned intrinsics and stereo extrinsics. A misclick by an operator could replace a working factory cal with a half-baked custom one and silently degrade the controller. Gating these behind the CLI keeps them available to advanced users without surfacing the foot-gun.

The stewart_vision calibration ROS node therefore implements ONLY Stage C service handlers:

- /calibrate/capture_frame (Trigger) — grab one frame of the leveled platform’s ArUco ring; auto-detects the board and returns success (“ArUco ring detected, all 8 markers visible”) or failure (“only 5 markers visible — check lighting and angle”).

- /calibrate/solve (Trigger) — cv2.aruco.estimatePoseBoard over the accepted frames; writes oak_extrinsics.yaml if the reprojection gate passes.

- /calibrate/reset (Trigger) — clears the captured-frame buffer.

There is no /calibrate/stage selector exposed via ROS — Stages A/B literally cannot be triggered from the GUI even if someone malicious typed the right service name. They only run when the calibrate_oak.py CLI is invoked on the Pi with the explicit flag.

The GUI’s Vision Debug panel shows the live OAK feed with detected ArUco markers drawn in real time, plus the three buttons above. The marker geometry comes from marker_layout.yaml produced by generate_aruco_ring.py so reprinting the ring doesn’t require code edits.

scripts/calibrate_oak.py runs in three independent stages so that moving the camera only invalidates the cheap stage. Stages A and B are optional now (v6 default): the OAK ships with factory-calibrated intrinsics and stereo extrinsics stored on its EEPROM, accessible via Device.readCalibration(). The default flow uses those directly:

# Default — write factory cal into oak_intrinsics.yaml in seconds
python3 scripts/calibrate_oak.py --stage factory

Re-doing Stages A/B manually only buys you a slightly tighter comparison-vs-truth baseline (your own chessboard captures may out-perform the factory <0.3 px reprojection target by a small margin). For the demo, use factory; document this in the report.

Stage C (camera-to-world via the ArUco ring) is always done ourselves — the OAK has no idea where the platform is.

### Stage A — Intrinsics, per eye (slow; do once per camera, ever)

- Print a 9×6 chessboard, 25 mm squares, mount on a rigid flat backer.

- Capture ~30 left+right pairs at varied angles and distances.

- Run cv2.calibrateCamera per eye → K_L, K_R, dist_L, dist_R.

- Acceptance: per-eye reprojection error < 0.3 px. Else recapture.

Outputs: oak_intrinsics.yaml (K, dist for each eye). These are the camera’s permanent properties; Stage A is rarely repeated.

### Stage B — Stereo extrinsics (do once per camera, ever)

- Same capture set as Stage A.

- cv2.stereoCalibrate(..., flags=CALIB_FIX_INTRINSIC) → R_LR, t_LR.

- cv2.stereoRectify → projection matrices P_L, P_R; rectification maps cached to disk.

- Acceptance: mean epipolar error after rectification < 1 px.

- Render an HTML report (calibration_report.html) showing rectified pairs with horizontal epipolar lines drawn on. This is the artifact the professor sees.

Outputs: appended to oak_intrinsics.yaml. Also rarely repeated.

### Stage C — Camera-to-world extrinsics (re-run after every camera move)

The articulated camera arm has no precise positioning, so this stage does not rely on a known camera pose. Instead it recovers the camera’s pose from the ArUco ring itself, which is rigidly attached to the top plate at known geometry (8 markers, 50 mm side, 120 mm ring radius, equal angular spacing — marker_layout.yaml). That gives 32 known 3-D corner positions in the platform frame.

- Manually level the platform: command zero tilt, confirm IMU at 0/0 ± 0.1° (matches §2 of the IMU discussion in chat history).

- Capture one frame with the full ArUco ring in view of the left eye.

- Run cv2.aruco.estimatePoseBoard (or equivalently solvePnP over all 32 corners) using marker_layout.yaml for the 3-D positions and K_L, dist_L from Stage A for the camera intrinsics.

- Because the platform is leveled and the ArUco ring is centered on the plate (which sits over the world origin), the recovered transform IS T_cam_world directly — no offset composition needed.

- Acceptance: project all 32 ring corners back into the image; mean reprojection error < 2 px. Below 1 px is the goal.

Output: oak_extrinsics.yaml. This file goes stale every time the camera arm moves. The calibrate script auto-detects whether Stage A/B outputs already exist and skips them — so re-extrinsics is ~30 seconds of work.

Why this works without preset support: the ArUco ring’s known geometry over-determines the pose problem (32 correspondences vs 6 DOF), so OpenCV’s solver converges robustly from any camera angle that sees ≥ 3 markers. Camera height, tilt, and azimuth are all recovered from the marker warping in a single frame. Mount the arm wherever; the math figures out the rest.

### Calibration sanity gate

calibrate_oak.py refuses to write its YAML if any acceptance metric fails, and prints which stage failed. Mis-calibrated extrinsics are the single most common cause of “the ball is in the wrong place” bugs; gating prevents bad data from ever entering the pipeline.

## 7. Stereo / vision techniques from Lecture 6

Strategy (locked in v5, 2026-04-26): the OAK’s onboard stereo depth is treated as ground truth, and the controller closes the loop on that. The lecture techniques (homography, F/E matrices, 8-point + RANSAC, triangulation, SfM) all run in parallel as comparison estimators only — they have no control authority. Each technique writes its output to a dedicated topic, and a comparison artifact (JSON + the per-demo HTML report from §13.5) shows agreement / disagreement between the OAK truth and each classical method.

Why: this is the highest-probability path to a working demo while still showcasing every Lecture 6 technique. If a classical estimator has a bad frame, the controller is unaffected (it’s looking at the OAK truth). If a classical estimator agrees with the OAK to within a few mm across the demo, that’s a strong “I understand this” claim to the professor — a single-frame disagreement plot is worth more than a flawless real-time estimator that anyone can run from a DepthAI tutorial.

Each item below is implemented explicitly somewhere in the pipeline.

| Lecture topic | Implementation in this project |
| --- | --- |
| Pinhole model + intrinsic K (slides 13–15) | oak_intrinsics.yaml; the visualization helper draws K’s principal point + focal-length cones. |
| Projection matrix P = K[R¦t] (slide 16) | Used in mono ball-pixel-to-platform-plane projection (ball_localizer_node.py). |
| Homography + 4-point + SVD (slides 17–22) | ArUco-board pose recovery (each marker → 4 correspondences). Also used internally by cv2.stereoRectify. |
| Checkerboard calibration (slides 23–25) | Stage A above. Calibration report shows the chessboard reprojection. |
| Stereo baseline + disparity (slides 26–27) | StereoSGBM disparity map computed on the OAK; published at low rate; rendered in the GUI debug pane. |
| Stereo rectification (slide 28) | Stage B; epipolar overlay in the report. |
| Epipolar geometry / F & E (slides 29–33) | epipolar_overlay.py draws horizontal epipolar lines on rectified pairs as a visual proof. |
| 8-point + RANSAC (slides 34–36) | sfm_demo.py recovers F from feature matches and confirms it agrees with factory-calibrated F (within RANSAC noise). |
| Triangulation (slide 40) | Primary stereo ball estimator — DLT triangulation of (uL, uR) → (X, Y, Z)_C for ball_xy_stereo. |
| Cheirality + R,t from E (slides 38–39) | Done in sfm_demo.py. |
| SfM pipeline (slide 41–42) | sfm_demo.py: pan camera by hand, recover camera trajectory via SIFT/ORB → F → E → triangulation. Standalone demo for the slides. |

Three ball-localization paths run in parallel; only /ball_xy_oak has control authority:

- OAK onboard depth (truth) (/ball_xy_oak) — ball pixel + the OAK’s StereoDepth output sampled at the ball ROI via DepthAI’s SpatialLocationCalculator. This is what ball_kf_node consumes and what the controller closes on.

- Mono + plane projection (classical) (/ball_xy_mono) — ball pixel + ArUco plane → ray-plane intersection. Pure Lecture 6 math (no stereo). Logged for comparison.

- Stereo triangulation (classical) (/ball_xy_stereo) — rectified pair → ball pixel in each → DLT triangulation → into platform frame. Uses our own intrinsics + 8-point fundamental matrix verification. Logged for comparison.

/ball_xy_oak ↔ /ball_xy_mono ↔ /ball_xy_stereo agreement is written to /method_comparison (custom JSON-encoded String at 30 Hz) and rolled up post-run by the plotter into the comparison section of the demo’s HTML report (§13.5). Per-frame error vs OAK truth (mm), Bland-Altman plots, and a histogram of disagreements per method.

### Other OAK-on-chip computations we shadow with Lecture-6 techniques

Beyond the ball-position estimators above, the OAK does several other things on-chip that we can replicate manually with classical methods. Each goes in the comparison artifact as another “we built it ourselves even though the camera does it” line. Recommended tier indicates implementation cost vs Lecture-6 demonstration value.

| OAK on-chip | Manual equivalent | Recommended? |
| --- | --- | --- |
| Stereo rectification (factory maps) | cv2.stereoRectify from our (or the factory’s) K + R + T → our own rectification maps; render both left/right pairs side-by-side with horizontal epipolar lines drawn (Lecture 6 slide 28) | Yes — one-shot, big visual proof, ~50 lines |
| Disparity (SGBM on Myriad X, real-time) | cv2.StereoSGBM_create on the host applied to the OAK’s rectified pair; per-pixel difference heatmap | Yes — a debug pane in the GUI is a great slide |
| Depth from disparity (Q matrix on-chip) | depth = (f · baseline) / disparity per-pixel using our own Q from cv2.stereoRectify; agreement scatter | Optional — implicit in disparity comparison |
| 3-D point at ROI (SpatialLocationCalculator) | Sample our own depth map at the ball pixel, project via K^-1 · pixel · depth; this is /ball_xy_stereo already | Already in the plan |
| Camera intrinsics (factory K, dist) | Stage-A chessboard cv2.calibrateCamera; compare K element-wise + reprojection error histograms | Optional (skipped for v6 default); only do if Stage A captures happen for some other reason |
| Stereo extrinsics (factory R, t) | cv2.stereoCalibrate after our own intrinsics; compare baseline length and rotation Euler angles | Optional; tied to Stage B |

Fallback story (extreme failure scenarios). If the OAK’s depth fails for some reason but raw frames keep flowing (e.g. stereo mismatch in heavy IR contamination), the classical mono ray-plane estimator (/ball_xy_mono) can take over control authority — same node already runs, just toggle which topic feeds ball_kf_node. The ArUco-ring pose recovery is independent of any depth path, so that survives. If the OAK device dies (USB drop, firmware lock-up), nothing recovers — the watchdog drops to LEVEL_HOLD; the operator power-cycles the OAK.

## 8. Ball detection and tracking

### Detection — onboard the OAK VPU, two detectors in parallel

Both detectors run simultaneously from day one (decision: 2026-04-25). Running them in parallel gives us cross-checking diagnostics and graceful degradation if either fails in some lighting condition.

- Color threshold (V0) — saturated-orange foam ball → HSV → threshold → biggest contour → centroid + radius. ~1 ms on the Myriad X. Zero training. Strong baseline given a bright orange ball on a black-and-white platform; light invariance is its main weakness.

- YOLOv5-nano (V1) — runs on the same Myriad X VPU at >30 fps. Trained on ~600–700 hand-labeled frames captured during a 5–6 minute free-roll session. scripts/capture_training_frames.py bags color frames at 2 Hz with timestamps and writes them as numbered JPEGs into a dataset directory; labels are drawn afterward in LabelImg, Roboflow, CVAT, or any YOLO-compatible tool. YOLOv5-nano is the natural fit: pre-quantized OpenVINO models exist; minimal training compute (a CPU-only train run takes ~30 min for this dataset size). Larger dataset = better generalization to varied lighting, glare from the IR projector, and partial occlusion by the operator’s hand.

Both detectors publish independently:

- /oak/ball/v0/left_pixel, /oak/ball/v0/right_pixel (color)

- /oak/ball/v1/left_pixel, /oak/ball/v1/right_pixel (YOLO)

ball_localizer_node subscribes to both. Selection rule:

- If both report a detection within 5 px and confidence ≥ 0.8 each, use the average → highest-quality measurement.

- Else if exactly one reports confidence ≥ 0.8, use it.

- Else if both report confidence < 0.8 but their detections agree within 10 px, use the average (low-confidence consensus).

- Else flag “ball lost” — KF predicts forward; if no acceptable detection for > 500 ms, drop to LEVEL_HOLD.

Each frame’s per-detector position + the chosen-output is published on /oak/ball/diagnostic so the GUI’s vision-debug strip shows live agreement / disagreement, telling you which detector is winning under the current lighting. This is itself a strong “we did the work” slide.

The Pi never decodes raw frames for tracking — only for the GUI live feed (compressed, 15 fps).

### Tracking — Kalman filter on the Pi

ball_kf_node.py runs a constant-velocity KF with state x = [px, py, vx, vy] (platform frame). This is exactly the bounding-box tracker derivation from Lecture 8 slides 32–34.

- Predict (100 Hz, fast loop): x ← F·x; P ← F·P·Fᵀ + Q.

- Update (~60 Hz, when measurement arrives): innovation y = z − H·x; S = H·P·Hᵀ + R; K = P·Hᵀ·S⁻¹.

- Q tuned from rolling-friction acceleration variance.

- R from empirical projection noise (estimate during calibration: park the ball, log 200 measurements, fit a 2-D Gaussian).

Optional EKF upgrade (Lecture 8 slides 35–37) — fold the tilted-plate dynamics directly into the motion model: ẍ = (5/7)·g·sin(θ_x) (foam, solid sphere; see §9). This makes Q smaller and predictions accurate during sensor dropouts. Worth doing if Demo 2 settling is choppy with the constant-velocity KF.

The KF posterior (px, py, vx, vy, P) is published on /ball_state. The controller subscribes to that — never to raw pixels.

## 9. Controller — PID with foam-ball feedforward

Ball-on-plate is a double integrator in each axis: tilt commands acceleration, position is the double-integral of acceleration.

- Pure P feedback on position is marginally stable.

- PI feedback is unstable (a third integrator in the loop).

- PID (P with derivative damping) is the minimum stable architecture.

- Adding a small I term compensates rolling-friction stiction at low speed.

### Plant equation (rolling ball)

For a sphere on a tilted plane, neglecting slip:

ẍ = (g · sin θ_x) / (1 + I/(m·R²))

The ratio I/(m·R²) differs by ball type:

| Ball | I/(m·R²) | ẍ as fraction of g·sin θ |
| --- | --- | --- |
| Hollow ping-pong (thin shell) | 2/3 | 3/5 = 0.600 |
| Solid foam (uniform density) | 2/5 | 5/7 ≈ 0.714 |

Foam accelerates ~19 % faster than ping-pong at the same tilt.

### Foam vs ping-pong — control trade-off

You purchased a 40 mm foam ball mass-matched to a ping-pong ball for its uniform mass distribution. From a controls standpoint:

- Ping-pong (hollow) — slower response → more time for the PID to react → easier to tune, more forgiving gain windows. But hollow + light makes it perturbed by air currents (fan, breath), and it dents on impact.

- Foam (solid) — faster response → tighter timing requirement, needs more aggressive Kd. But mass distribution is uniform (no off-center weight from a manufacturing seam), more durable, less air-perturbed, repeatable across thousands of runs.

Recommendation: keep both. Use the ping-pong for initial PID tuning (gentler dynamics → faster convergence to stable gains), then swap to the foam for the final demo (more visually crisp orbital motion, more reliable repeatability). Same vision pipeline either way (both 40 mm, both bright orange).

The ball_safety.yaml file (§10) carries a ball_density field — solid or hollow — so swapping balls just changes one config line and the feedforward coefficient updates automatically.

### Control law (per axis, x and y identical)

e   = x* − px
edot= ẋ* − vx               // KF gives vx
α   = 5/7 if solid else 3/5  // from ball_safety.yaml
θ_x = Kp·e + Kd·edot + Ki·∫e + (1/(α·g))·ẍ*    // last term is feedforward

Tune order: Kp first (Kd=Ki=0, watch oscillation), add Kd to damp, add small Ki for stiction. Then verify the feedforward term reduces tracking error at high orbital speeds (Demo 1 is the test case).

### Output handling

- Saturate θ_x, θ_y to whatever the platform achieves smoothly, enforced by the existing global_limits.yaml.

- Slew-rate limit on tilt commands (~30°/s) to prevent the Stewart legs from chattering.

- Roll/pitch only — never yaw — because the ArUco ring rejects yaw estimation noise more cleanly anyway.

### Z-height handling

The Stewart platform can translate the top plate vertically as well as tilt it. Demo 2’s z-height slider commands a constant z-offset added to all six leg lengths (no tilt, just up/down). This is independent of the ball-tracking PID, but the controller must be told the current plate-z so the ArUco-pose-based ball projection uses the right plane height. Confirm this is the intended slider behavior — see §15 Q1.

## 10. Safety dead-zone (10 mm rule)

Constraint: the ball’s surface must stay ≥ 10 mm from the platform geometric center to clear the IMU bolt.

Ball radius = 20 mm (40 mm foam ball) → ball center must stay at radius ≥ 30 mm from platform center.

config/ball_safety.yaml:

dead_zone_mm: 10        # ball-edge to platform-center clearance
ball_radius_mm: 20      # solid foam, 40 mm dia
ball_density: solid     # affects feedforward (5/7 vs 3/5 of g)
margin_mm: 5            # soft-repulsion starts here outside dead-zone
loss_timeout_ms: 500    # fall back to level if ball not detected

Three independent enforcement layers (matches the existing global_limits.yaml redundancy philosophy):

- Reference generator — any reference point with √(x*² + y*²) < dead_zone + ball_radius is rejected:

  - Demo 1: orbit radius silently clamped to ≥ 35 mm.

  - Demo 2: clicks inside the red disk flash and are ignored.

  - Demo 3: any path sample inside the dead-zone fails the pre-flight check and the entire path is rejected (operator must redraw — silent clipping was considered and rejected; a redrawn path is more honest than a silently-mutated one).

- Controller — if the measured ball is within dead_zone + ball_radius + margin of center, add a soft repulsive tilt term that accelerates the ball outward. This kicks in before the ball can reach the IMU bolt and continues even if the reference would otherwise drive it inward.

- Watchdog — center incursion — if the measured ball radius falls below dead_zone + ball_radius for > 100 ms, command level + slow z-down and fault out. Recovery requires operator clear via GUI.

- Watchdog — off-platform — if the measured ball position has √(x² + y²) > 200 mm (i.e. outside the platform’s physical radius) or both detectors report ball-lost for > 500 ms, abort to LEVEL_HOLD immediately. This is the only condition that triggers the strobing red ball-lost UI flash (§11.5) — transient in-bounds detection dropouts (e.g. the operator’s hand briefly shadowing the ball) are recovered silently by the controller. Re-arming is manual: the operator must press Start again after the ball is back inside a 180 mm hysteresis radius and detected with confidence ≥ 0.8 for 1 s. Manual is intentional — automatic re-arm during a “hand still on the platform” moment would be surprising.

The graphical disk in the Demo-2 / Demo-3 GUI draws the dead-zone in semi-opaque red so the operator can’t accidentally click or draw into it.

## 11. GUI design

The GUI remains a single web/index.html Tailwind + roslibjs page served by gui_server.py. The page now uses a three-column layout to keep all live information visible at once.

### 11.0 Three-column layout (already in web/index.html)

The current web/index.html is already three-column on lg: breakpoint and above (<main class="grid lg:grid-cols-3"> with three <section class="lg:col-span-1"> children). On screens narrower than 1280 px the columns stack vertically. The vision-driven demos add panels into the existing layout rather than restructuring it:

┌──────────────┬──────────────────────────────┬──────────────┐
│   LEFT       │           MIDDLE              │   RIGHT      │
│              │                               │              │
│  + Vision    │  + Demo 1 (Orbit)             │  Homing      │
│    Debug     │  + Demo 2 (Goto)              │  Jog         │
│  + Calib     │  + Demo 3 (Path) [stretch]    │  Level       │
│    Stage C   │   each with Start / Stop +    │  Reset stack │
│    only      │   graphical SVG disk view     │  (existing)  │
│              │                               │              │
│  Encoder &   │  (Open-loop "rolling ball"    │              │
│  current     │   panel hidden by default     │              │
│  debug       │   when vision is active)      │              │
│  (existing)  │                               │              │
└──────────────┴──────────────────────────────┴──────────────┘

+ = NEW panels added by the vision work. Existing panels keep working unchanged.

### 11.1 Left column — Vision Debug + existing diagnostics

Vision Debug (top of left column, collapsible):

- Live left-eye feed (compressed, 15 fps, <canvas> from a sensor_msgs/CompressedImage topic).

- Live right-eye feed, stacked under or beside the left depending on column width.

- Toggle: “Rectified” overlays horizontal epipolar lines (visual proof of Lecture 6 slide 28).

- Toggle: “Disparity” — replaces feeds with the StereoSGBM color-mapped disparity map at low rate.

- Telemetry strip:

  - ArUco markers visible (n / 8)

  - Tracker confidence (KF NIS, color-coded)

  - End-to-end latency (stamp_now − stamp_capture)

  - Method comparison (live, mm): OAK-vs-mono, OAK-vs-stereo, mono-vs-stereo discrepancies. Color-coded:

    - green when |error| < 5 mm

    - yellow when 5 mm ≤ |error| < 15 mm

    - red when |error| ≥ 15 mm

Calibration controls (collapsible sub-panel, hidden during demos):

This panel exposes Stage C only — the ArUco-ring camera-to-world step that’s re-run after every camera arm move. Stages A/B are CLI-only by design (see §6).

- Live OAK left-eye feed with detected ArUco-ring corners drawn in real time (so the user can see whether the current pose has all 8 markers visible before pressing Capture).

- Frame counter “N captured” — typically 1–3 frames is enough for Stage C with a well-leveled platform; the solver accepts a single frame as long as ≥ 3 markers are detected.

- Three buttons (only Stage C operations exposed via ROS):

  - Capture frame — calls /calibrate/capture_frame. Result toast: “all 8 markers detected” / “only N/8 markers visible — rotate camera or improve lighting”.

  - Solve — calls /calibrate/solve. Runs cv2.aruco.estimatePoseBoard over the accepted frames; writes oak_extrinsics.yaml if mean reprojection error < 2 px.

  - Reset — clears the buffer.

The GUI does not offer a stage selector. Operators cannot override the OAK’s factory intrinsics or stereo extrinsics from the browser — those are protected behind the CLI flow on the Pi to prevent accidental damage to a working calibration.

Encoder & current debug (bottom of left column): the existing plots and read-outs, unchanged.

### 11.2 Middle column — Demo panels

Each demo panel contains its own SVG disk view (so the operator never has to scroll between controls and the visualization), plus mode- specific controls. Only one demo can be active at a time; activating one disables the others’ Start buttons.

#### 11.2.1 Demo 1 panel — Orbit

- Orbit radius slider (35 mm to platform_radius − ball_radius − 5).

- Orbit period slider (1 to 10 s).

- Direction toggle (CW / CCW).

- Reverse Direction button — visually styled like the UNO “reverse” card: two curved arrows chasing each other in opposite directions, bright yellow background. Pressing it flips the orbit direction while running — the reference reverses instantly and the controller commands the appropriate tilt to decelerate then accelerate the ball the other way (great visual stress test of the loop’s responsiveness; matches lecture-8 PID material). Disabled when not orbiting.

- Start button: publishes mode change to BALL_TRACK_TRAJECTORY; the controller first runs the goto law to drive the ball to (R, 0), then engages the orbit reference.

- Stop button: drops to LEVEL_HOLD.

- Mini-SVG disk: shows reference orbit circle + measured ball trail.

#### 11.2.2 Demo 2 panel — Click-to-position

A square <svg> showing a top-down platform representation:

- Outer circle: platform_radius (200 mm, drawn 1:1 in mm at the user’s zoom).

- Dotted circle at 120 mm: ArUco ring (with the 8 marker squares drawn to scale, labeled 0–7 to match marker_layout.yaml).

- Solid red filled disk: dead-zone (no-go region for both clicks and ball).

- Filled blue dot: current ball position from /ball_xy (platform frame).

- Hollow blue ring: current ball reference from /ball_ref (only drawn while in BALL_TRACK_GOTO mode).

- Predicted-path overlay: dashed line from current ball position to target, controlled by a “Show predicted path” checkbox (default off).

- Faint trail: last 2 seconds of ball positions (debug toggle).

Click handler: any click inside the platform-but-outside-dead-zone publishes a new /ball_ref and switches mode to BALL_TRACK_GOTO. A second click while a goto is in flight cancels the current target and retargets immediately. Clicks inside the dead-zone flash the red region and ignore the input.

Auxiliary controls:

- Start button: arms goto mode (clicks become live targets). Without Start armed, clicks are previewed on the SVG but do not publish.

- Stop button: clears /ball_ref, drops to LEVEL_HOLD, disarms goto.

- Z-height slider: commands pure heave on the Stewart top plate (no tilt change). Range 0 to 80 mm above neutral pose. Updates take effect immediately while the demo is running.

- “Cancel target” button: clears /ball_ref without disarming Start.

- Mode indicator: LEVEL_HOLD / BALL_TRACK_TRAJECTORY / BALL_TRACK_GOTO / BALL_TRACK_PATH.

#### 11.2.3 Demo 3 panel — Path drawing & follow (stretch)

Same SVG disk as Demo 2, plus:

- Draw mode toggle: while on, mouse drag draws a freehand path (one (x_mm, y_mm) sample every ~30 ms during drag).

- Path display modes (toggle inside the panel):

  - (b) Trail-erase mode (default) — the segment behind the ball greys out as the ball passes, giving an unmistakable “you are here” cue along the polyline. Easiest to read at a glance.

  - (a) Static green — entire polyline stays bright green for the whole traversal, no progress indication.

- The drawn path renders as a green polyline during draw; segments that pass through the dead-zone or off-platform render in red so the operator sees the violation immediately.

- Loop checkbox: closes the path with a cubic-spline join end→start and traverses repeatedly.

- Traversal speed slider (10 to 200 mm/s, default 80) — operator-set, not automatic.

- Predicted direction checkbox: shows arrows along the polyline indicating traversal direction (faint when off-current-segment, bright on the current segment).

- Reverse Direction button (UNO-reverse style, same yellow curved-arrows look as Demo 1) — flips traversal direction at the ball’s current path-position (no jump back to the start). With Loop on, the looping closure direction also flips. With Loop off, reaching either endpoint of the polyline ends the run and drops to LEVEL_HOLD. The trail-erase state resets at each reverse so the display reflects “where the ball has gone since the last reverse” rather than carrying stale greying. Disabled when not running.

- Clear button: erases the drawn path.

- Start button: validates the path against safety rules (any red segment → reject with toast), drives the ball to the path’s start via goto law, then engages BALL_TRACK_PATH mode.

- Stop button: drops to LEVEL_HOLD. The drawn path is preserved so Start can be pressed again without redrawing.

##### Path persistence

- Browser localStorage with named save/load slots (e.g. “figure eight”, “spiral in”, “demo day path”). Slots persist across page reloads and across browser sessions on the same machine.

- Export to file — a “Download .json” button serializes the active path to a JSON file:

- {
  "schema_version": 1,
  "frame": "platform",
  "units": "mm",
  "loop": false,
  "speed_mm_s": 80,
  "samples": [[x0, y0], [x1, y1], ...]
}

- Import from file — an “Upload .json” button reads the same format and loads it as the active path. The schema_version field lets future format changes coexist with old files.

##### Disturbance recovery

If the ball is bumped off the path mid-traversal, the controller’s behavior depends on where the ball ends up:

- Ball still on platform (within 200 mm of center): retarget to the closest point on the polyline and continue from there. No abort.

- Ball off platform (radius > 200 mm) or detected lost > 500 ms: abort to LEVEL_HOLD. This is the “operator picked it up to do something else” path. Re-arm via the path’s Start button when ready.

### 11.3 Right column — Existing controls

The right column hosts the existing operator controls unchanged:

- Homing routine

- Jog leg

- Level platform

- Reset stack (the gui_server.py HTTP endpoint, useful when rosbridge is unreachable)

- Open-loop “Demo: rolling ball” panel — kept for fallback / regression testing only; hidden by default when vision is active, expandable from a small toggle.

### 11.4 Cross-cutting controls

Above the three columns, a thin status bar:

- Connection state (rosbridge, OAK, control node)

- Active demo + mode indicator

- Big red Emergency Stop button (calls e_stop service, drops legs, levels platform). Always visible.

### 11.5 Ball-lost visual feedback

The strobe fires only when the off-platform watchdog (§10.4) trips — that is, the ball physically falls off the platform or both detectors lose it for > 500 ms together. In-bounds disturbance recovery (Demo 3 retargeting the closest path point, transient occlusion, etc.) is silent — no strobe, just the controller working.

When the strobe condition is met:

- The page’s <body> background colour strobes between neutral and red at ~3 Hz for 3 seconds. The strobe affects only the body background, not panel/card backgrounds and not any overlay. All controls stay fully readable and clickable throughout — the goal is to alert the eye, not block the GUI.

- After the strobe, the body background settles on a dim red tint that persists until the ball is detected again with confidence ≥ 0.8 for 1 s (matches the re-arm condition in §10.4) AND the operator presses Start (manual re-arm).

- A toast message (“Ball lost — platform leveling. Replace ball on platform and press Start to re-arm.”) appears top-center.

- The event is also logged silently to the standard ROS bag — the visual flash is in addition to, not instead of, the log.

- No audible beep (current decision; trivial to add later if desired).

The strobe is GUI-only; the platform itself goes to LEVEL_HOLD without delay regardless of the GUI state.

## 12. Topics, services, and messages

### New topics published by stewart_vision

| Topic | Type | Rate | Notes |
| --- | --- | --- | --- |
| /oak/left/image_compressed | sensor_msgs/CompressedImage | 15 Hz | GUI live feed only |
| /oak/right/image_compressed | sensor_msgs/CompressedImage | 15 Hz | GUI live feed only |
| /oak/disparity_compressed | sensor_msgs/CompressedImage | 5 Hz | Debug toggle |
| /oak/ball/left_pixel | geometry_msgs/PointStamped + confidence | ≤ 60 Hz | (cx, cy) in left image |
| /oak/ball/right_pixel | geometry_msgs/PointStamped + confidence | ≤ 60 Hz | (cx, cy) in right image |
| /platform_pose | geometry_msgs/PoseStamped | 30 Hz | ArUco board → camera frame |
| /ball_xy_oak | geometry_msgs/PointStamped | 60 Hz | Truth — OAK SpatialLocationCalculator output, platform frame. Controller closes on this. |
| /ball_xy_mono | geometry_msgs/PointStamped | 60 Hz | Classical mono ray-plane projection. Logged only, no control authority. |
| /ball_xy_stereo | geometry_msgs/PointStamped | 60 Hz | Classical 8-point + DLT triangulation. Logged only, no control authority. |
| /method_comparison | std_msgs/String (JSON) | 30 Hz | Per-frame OAK-vs-classical disagreements (mm) for live + post-run analysis. |
| /ball_state | custom (px, py, vx, vy, covariance) | 100 Hz | KF posterior fed by /ball_xy_oak. |
| /ball_ref | geometry_msgs/PointStamped | event-driven | Set by ref_generator_node |

### Calibration services (Stage C ONLY — A/B are CLI-only by design)

| Service | Type | Notes |
| --- | --- | --- |
| /calibrate/capture_frame | std_srvs/Trigger | One frame per click; rejects frames with < 3 ArUco markers visible. |
| /calibrate/solve | std_srvs/Trigger | Runs cv2.aruco.estimatePoseBoard, writes oak_extrinsics.yaml if reprojection error < 2 px. |
| /calibrate/reset | std_srvs/Trigger | Clears the captured-frame buffer. |

There is no stage selector service. Stages A/B (custom intrinsics and stereo recalibration) override the OAK’s factory tuning and therefore live behind the CLI only — see §6 for the rationale.

### New /control_cmd payloads (existing topic)

The GUI uses the existing /control_cmd String topic to switch modes (rosbridge service-discovery workaround):

- mode:LEVEL_HOLD

- mode:BALL_TRACK_TRAJECTORY + JSON params (radius, period, dir, phase)

- mode:BALL_TRACK_GOTO + JSON target (x_mm, y_mm)

- mode:BALL_TRACK_PATH + JSON params (path_id, speed_mm_s, loop)

- z_offset:<mm> for the z-height slider

stewart_control_node already routes /control_cmd; we just add the new payload patterns to its dispatcher.

### Path upload (Demo 3)

Path uploads are large enough that stuffing them into /control_cmd’s String is awkward. Use a dedicated topic:

- /ball_path/upload (nav_msgs/Path in platform frame, header.frame_id = “platform”) — published once when the operator presses Start in Demo 3. The reference generator stores the path under a UUID and returns it via /ball_path/active for the GUI to render the current-segment highlight.

## 13. Latency budget

End-to-end loop must close in ≤ 40 ms to avoid wobble:

| Stage | Target |
| --- | --- |
| OAK exposure + on-VPU detection | ≤ 15 ms |
| USB3 → Pi + ROS topic hop | ≤ 3 ms |
| ArUco pose + projection | ≤ 2 ms |
| KF predict/update | < 1 ms |
| Controller tick | ≤ 5 ms |
| Stewart leg command via CAN | ≤ 5 ms (already measured) |
| Total worst case | ≤ 31 ms |

Measure this with stamp_now − stamp_capture rolling average displayed in the Vision Debug telemetry strip. Tune only after the budget is green — tightening PID gains to fight latency hides the real fault.

## 13.5 Logging and post-run plotter (per-demo bag schemas)

Each demo records its own bag schema and ships with a tailored plotter in stewart_vision/scripts/plot_demo_run.py. The reasoning: the existing analyze_routine_log.py is great for the open-loop bring-up runs but lacks the vision-loop fields, and a single one-size-fits-all plot grid for the new demos would be cluttered and ugly. Per-demo plots impress.

### Common bag tracks (all demos)

| Topic | Type |
| --- | --- |
| /encoders | existing |
| /imu/data | sensor_msgs/Imu |
| /platform_pose | geometry_msgs/PoseStamped |
| /ball_state | custom (px, py, vx, vy, P) |
| /ball_xy_mono, /ball_xy_stereo | both paths |
| /oak/ball/diagnostic | per-detector positions + chosen output |
| /control_cmd | mode transitions |
| /oak/disparity_compressed (downsampled) | for the cover slide |

### Demo-1-specific tracks

| Topic | Notes |
| --- | --- |
| /ball_ref | current orbit reference |
| /orbit_params | R, T, direction, reverse events |

Plots produced (one PNG per panel, plus a summary HTML):

- Reference orbit + measured trajectory in platform frame, with the orbit circle drawn for context, colored by time.

- Radial error vs time: |√(px² + py²) − R|.

- Tangential / phase error vs time.

- Tilt commands (roll, pitch) and per-leg lengths vs time.

- Latency histogram (stamp_now − stamp_capture).

- V0 vs V1 detector agreement (scatter + Bland-Altman).

- Direction-reversal events overlaid as vertical bars on every plot.

### Demo-2-specific tracks

| Topic | Notes |
| --- | --- |
| /ball_ref | each click target as a transition |

Plots produced:

- Trajectory traces from each previous-position to each clicked target (one polyline per goto leg, colored by leg).

- Settling-time histogram across the run.

- Time-to-target vs Euclidean target distance scatter (regress for expected speed; outliers are interesting).

- Tilt commands vs time, segmented by goto leg.

- 2D error heatmap: where on the platform does settling take longer?

### Demo-3-specific tracks

| Topic | Notes |
| --- | --- |
| /ball_path/active | the validated polyline being followed |
| /path_params | speed, loop flag, reverse events |

Plots produced:

- Reference path vs measured trajectory, ball trajectory colored by along-track progress.

- Cross-track error (perpendicular distance from path) vs time.

- Along-track progress vs time (should be linear in the constant-speed regime — slope deviations show where the ball lagged or led).

- Per-loop overlay if Loop was on (lap 1 vs lap 2 vs … in different colors) — repeatability is the wow plot here.

- Reverse events as vertical bars; trail-erase resets visible.

### Method-comparison section (added v5, every demo’s report)

Drives the “we implemented every Lecture 6 technique” claim. From /method_comparison (JSON @ 30 Hz) and the per-method ball-position topics:

- 2-D scatter: OAK truth (x) vs each classical method (y), per axis, with the y = x line drawn. Tight clustering = method works.

- Bland-Altman plot per classical method: mean of (truth, classical) on x-axis, difference on y-axis; mean ± 1.96σ horizontal lines. The standard way to visualize agreement between two estimators.

- Per-method statistics table:

  - mean error vs truth (mm)

  - RMS error vs truth (mm)

  - 95th-percentile error (mm)

  - frames where the method produced no estimate (failure rate)

- Time-series of per-method error during the run, with demo events (orbit reverse, goto target reached, path waypoints) overlaid as vertical bars.

This is the artifact that proves to the professor that every classical technique was implemented correctly, even though we chose the OAK’s hardware-accelerated depth for robust real-time control. Also identifies which classical method would have been the viable fallback had the OAK failed — useful “what if” answer for any “why didn’t you use stereo triangulation directly?” question.

### Plotter interface

ros2 run stewart_vision plot_demo_run \
    --bag <bag_dir> --demo {1|2|3} --out <out_dir>

Reads the bag once, classifies events, and emits three artifact families under <out_dir> (decision: 2026-04-26):

- index.html — browser-friendly summary with embedded thumbnails, per-section nav, and inline summary stats. The artifact you take into the review meeting.

- panels/*.png — full-resolution per-panel images so you can pull individual plots into a slide deck or a paper without parsing the HTML.

- comparison.csv — one row per frame, columns for OAK truth (x, y, z), each classical method (x, y, z), per-axis errors, and the chosen-method tag. Easier for grading scripts, Excel pivots, or feeding into another notebook.

Plus a summary.json with the rolled-up statistics (RMS errors, settling-time medians, lap repeatability, latency p50/p95) for machine consumption.

Latency, fault, method-comparison, and detector-agreement panels are common to all three demo types so the boilerplate is shared in _common_panels.py.

## 14. Implementation milestones

Each milestone has an explicit “DONE WHEN” gate, in the same style as bringup_plan.md.

- Print and stick the ArUco ring. DONE WHEN: visual inspection confirms 8 markers at 120 mm radius, matte sticker is flat (no bubbles), no glare under IR projector.

- OAK driver up. DONE WHEN: /oak/left/image_compressed shows a stable image in the GUI. Frame timestamps monotonic; drop rate < 1 %.

- Calibration Stage A + B done; report generated. DONE WHEN: oak_intrinsics.yaml written; calibration_report.html shows < 0.3 px reprojection / < 1 px epipolar; visual inspection of rectified pairs shows aligned epipolar lines.

- Calibration Stage C done. DONE WHEN: oak_extrinsics.yaml written; ArUco ring projects back into the image at < 2 px error. (This is repeated whenever the camera arm moves.)

- /platform_pose at ≥ 30 Hz. DONE WHEN: tilt the plate by hand → roll/pitch from /platform_pose agrees with the IMU within 1° static.

- Ball pixel detection. DONE WHEN: drop ball into frame; /oak/ball/left_pixel and right_pixel track it by eye; confidence > 0.8 in normal light.

- /ball_xy_mono in mm. DONE WHEN: place the ball at known offsets from center; reported value within 3 mm of ruler measurement at all four cardinal points and at platform_radius − ball_radius.

- /ball_xy_stereo agrees with mono. DONE WHEN: max disagreement < 5 mm across the platform.

- KF tracking. DONE WHEN: roll the ball by hand; /ball_state velocity matches the visible motion direction and magnitude (eyeball test); NIS between 0.5 and 5 most of the time.

- Closed-loop ball-centering (sanity). DONE WHEN: drop ball off-center, platform tilts to push it toward center; settles within ±10 mm of center inside 10 s. PID tuning starts here.

- Demo 1: orbit at fixed radius. DONE WHEN: ball orbits at requested R within ±5 mm RMS for one full minute at T = 4 s.

- Demo 2: click-to-position. DONE WHEN: click any 5 random points on the GUI; ball reaches each within ±10 mm and holds for 5 s before next click.

- Demo 2: dead-zone enforcement. DONE WHEN: clicks inside red zone are ignored; manually pushing the ball into the dead-zone triggers the soft-repulsion controller and the watchdog fault.

- Demo 3 (stretch): path follow, single-pass. DONE WHEN: draw a 5-vertex polyline; ball reaches start then follows path within ±15 mm RMS at 80 mm/s.

- Demo 3 (stretch): looped path. DONE WHEN: same path with Loop checked; ball completes 3 consecutive laps without RMS error degrading > 50 % from lap 1.

- Demo 3 (stretch): path safety. DONE WHEN: paths drawn through the dead-zone are rejected at the UI with a clear toast; paths drawn off-platform same.

- YOLO V1 training data captured and labeled. DONE WHEN: 5–6 min of free-rolling-ball capture run via capture_training_frames.py; 600+ frames labeled (orange ball bbox per frame); train/val split saved.

- YOLO V1 trained and deployed on the OAK. DONE WHEN: V1 publishes to /oak/ball/v1/*pixel at ≥ 30 fps; /oak/ball/diagnostic shows median V0–V1 disagreement < 5 px in normal lighting.

- Per-demo plotter shipping. DONE WHEN: plot_demo_run.py --demo {1|2|3} produces an HTML index + per-panel PNGs from a real bag for each demo type. Panels load in a browser with no missing data warnings.

- Off-platform abort + strobe. DONE WHEN: rolling the ball off the platform edge triggers LEVEL_HOLD within 200 ms, the GUI body background strobes red for 3 s, and Start re-arm only succeeds after the ball is back on-platform for 1 s.

## 15. Open questions

### Resolved (2026-04-25)

| # | Question | Decision |
| --- | --- | --- |
| Q1 | Z-height slider semantics | Pure heave, no tilt change, 0–80 mm above neutral. |
| Q2 | Live camera feed during Demo 2 | Stays in left-column Vision Debug panel, above the encoder/current debug values. |
| Q3 | Click during in-flight goto | Cancel and retarget immediately. |
| Q4 | Predicted-path overlay | Toggleable checkbox in each demo panel, default off. |
| Q5 | Demo 1 orbit start | Drive ball to orbit radius from current location via Demo-2 goto law, then engage orbit. |
| Q6 | Camera mount preset | Not possible (free-form articulated arm) — instead, recover camera pose from the ArUco ring’s known geometry (Stage C, §6). |
| Q7 | GUI layout | Three-column layout: left = Vision Debug + encoders, middle = demo panels, right = existing controls. |
| Q8 | Start / Stop buttons | Mandatory on Demo 1, Demo 2, AND Demo 3. Big red Emergency Stop above the columns. |
| Q9 | Demo 3 (path drawing) | Added as stretch goal. Polyline draw + smoothing + arc-length-resampled traversal + optional loop. |
| Q10 | Ball choice | Foam (40 mm, mass-matched, solid) for the final demo; tune PID with ping-pong first because gentler dynamics converge faster. |
| Q11 | Demo-1 direction reversal | Yes — UNO-reverse-card-style button, available mid-orbit. |
| Q12 | Path traversal speed | Operator-set slider, default 80 mm/s. |
| Q13 | Path mid-traversal disturbance | (a) retarget closest path point if ball stays on platform; (b) abort to LEVEL_HOLD if ball leaves platform (>200 mm from center) or detection lost >500 ms. 180 mm hysteresis radius for re-arm. |
| Q14 | Path persistence | Browser localStorage with named save/load slots plus export/import to .json file (schema_version 1, see §11.2.3). |
| Q15 | Path display during traversal | Default = trail-erase (segment behind ball greys out); toggleable to static-green-polyline. |
| Q16 | Path-into-dead-zone handling | Reject the whole path; user must redraw. No auto-clip. |
| Q17 | YOLO timing | Build V1 in parallel with V0 from day one; both publish, ball_localizer cross-checks (§8). |
| Q18 | Ball-lost UI feedback | Background strobes red ~3 Hz for 3 s, then dim-red until re-armed. Silent log also kept. No beep. |

Q19 | Logging format | Per-demo bag schemas + dedicated plot_demo_run.py plotter producing per-demo PNG panels and an HTML index — the wow artifact for review (§13.5). |
Q20 | YOLO training data capture | capture_training_frames.py records JPEGs at 2 Hz; plan calls for 5–6 min of free-rolling capture (~600–700 frames) for stronger generalization. |
Q21 | Re-arm after off-platform abort | Manual only. Operator must press Start once ball is back inside 180 mm hysteresis radius and detected for 1 s. |
Q22 | UNO-reverse button placement | Demo 1 panel and Demo 3 panel (path drawing). Not Demo 2; not the status bar. |
Q23 | Strobe-red trigger scope | Off-platform watchdog only. In-bounds disturbance recovery is silent. Strobe is body-background-only — never overlays panels or controls. |

### Resolved in v5 (2026-04-26)

| # | Question | Decision |
| --- | --- | --- |
| Q24 | Compute platform | Raspberry Pi 5, 16 GB RAM, ROS 2 Kilted. Phase 8 migration complete. systemd auto-launch. |
| Q25 | OAK USB port | Blue USB-A 3.0 on the Pi 5 (USB-C is power-only on Pi 5). |
| Q26 | Network | Same-LAN via Pixel 9 hotspot. GUI at http://<pi-ip>:8080/. Hostname stablebot. |
| Q27 | Vision strategy | OAK onboard depth = ground truth (controller closes on it). Lecture-6 classical methods run in parallel as comparison estimators only. |
| Q28 | Comparison artifact | Live /method_comparison JSON @ 30 Hz for the GUI’s vision-debug strip; rolled up post-run by plot_demo_run.py into a dedicated section of the demo HTML report. |
| Q29 | Calibration capture flow | GUI buttons (Capture / Solve / Reset) calling Pi-side ROS services — Pi is headless. |
| Q30 | Chessboard generator | Shipped (stewart_vision/scripts/generate_chessboard.py). |

### Resolved in v6 (2026-04-26)

| # | Question | Decision |
| --- | --- | --- |
| Q31 | Calibration shortcut | Default to --stage factory — read OAK EEPROM via Device.readCalibration() and write to oak_intrinsics.yaml. Stages A/B remain available for users who want a tighter custom baseline. |
| Q32 | Plotter outputs | HTML index + panels/*.png + comparison.csv + summary.json. CSV columns documented in §13.5 plotter section. |
| Q33 | Live KPI thresholds | green < 5 mm, yellow ∈ [5, 15) mm, red ≥ 15 mm — applied to all three method-comparison numbers in the vision-debug strip. |
| Q34 | Additional shadowed comparisons | Add custom rectification + custom SGBM disparity as recommended Lecture-6 demonstrations (§7 table). Custom intrinsics/extrinsics deferred (tied to Stages A/B which we’re skipping). |
| Q35 | Failure-mode fallbacks | OAK depth fail → toggle controller to /ball_xy_mono (already running, just no control authority by default). OAK device fail → watchdog to LEVEL_HOLD; operator power-cycles. |

### Resolved in v7 (2026-04-26)

| # | Question | Decision |
| --- | --- | --- |
| Q36 | GUI calibration scope | Stage C ONLY (camera-to-world ArUco ring). Stages A/B never reachable from the GUI — they override factory tuning and live behind the CLI flow on the Pi to prevent operator misclicks. |
| Q37 | Existing web/index.html layout | Already three-column (lg:grid-cols-3); the vision work adds panels into the existing structure rather than restructuring. |

### Still open

None — spec locked at v7 (2026-04-26).

## 16. References

- stewart_bringup/docs/NEXT_STEPS.md — prior vision plan (this doc refines it).

- bringup_plan.md — master 8-phase bring-up plan.

- stewart_bringup/scripts/generate_aruco_ring.py — produces the printable ArUco ring + marker_layout.yaml.

- stewart_bringup/config/global_limits.yaml — three-layer speed cap redundancy pattern (mirrored for the ball dead-zone).

- stewart_bringup/config/platform_level.yaml — IMU-frame level reference (now zeroed against the MTi factory cal as of 2026-04-25).

- EEL 4930/5934 Lecture 6 — Robot Perception (camera model, homography, calibration, stereo, epipolar geometry, F/E matrices, 8-point + RANSAC, triangulation, SfM).

- EEL 4930/5934 Lecture 8 — Localization, Filtering & State Estimation (Bayesian filter, KF, EKF, PID).

- Movella MTi 600-series User Manual — IMU sensor-frame conventions.

- OpenCV cv2.aruco, cv2.calibrateCamera, cv2.stereoCalibrate, cv2.stereoRectify, cv2.solvePnP, cv2.triangulatePoints.

- Luxonis DepthAI documentation (OAK-D-PRO-AF pipeline).