# Stable-Bot Stewart Platform

Browser-controlled 6-DOF Stewart platform with dual-IMU closed-loop

leveling, keyframe routine playback, and data logging. Built on ROS 2

Kilted, six ODrive Pro motor controllers over CAN, and two Xsens MTi-630

IMUs (base + platform).

Live pilot UI: http://localhost:8080/ once the stack is up.

— — — — —

## What's in this package

`stewart_bringup/stewart_control_node.py` — the one long-lived ROS 2

node. Owns the CAN bus via a per-leg-mode ODriveFeeder (50 Hz

Set_Input_Pos with velocity feed-forward), passive + RTR

EncoderListener for encoders / errors / Iq, PI level loop, homing

subprocess orchestrator, routine playback + data logging. All GUI

actions reach it via a /control_cmd topic dispatcher (see

[ARCHITECTURE.md](docs/ARCHITECTURE.md)).

`web/index.html` — single-file web GUI, roslibjs + Tailwind via CDN.

`launch/stewart_gui_launch.py` — bundled launch: dual Xsens IMUs +

control node + rosbridge on ws://localhost:9090.

`scripts/gui_server.py` — tiny HTTP server that serves the HTML **and**

exposes /reset, /launch, /stop_launch, /launch_status

endpoints so the browser can recover the backend when rosbridge is

unreachable.

`scripts/reset_stewart_stack.py` — one-shot recovery: kills stale

processes, frees port 9090, restarts the ros2 daemon, optionally

relaunches + polls /status to verify the stack is actually

publishing.

`scripts/analyze_routine_log.py` — offline analyzer for recorded

routines; plots commanded vs measured tilt, leg currents, and runs a

rolling-sphere ball-on-disk simulation against the measured IMU data.

`scripts/read_odrive_errors.py` — RTR query of every ODrive's

active_errors + disarm_reason with human-readable decode.

— — — — —

## Hardware inventory

| Item | Interface | Role |
| --- | --- | --- |
| 6 × ODrive Pro (firmware 0.6.x) | CAN (gs_usb adapter) | leg motor drives |
| 2 × Xsens MTi-630 | USB-serial at 921600 baud | base + platform IMUs |
| 6 × 3D-printed linear actuators (~71 mm / motor rev) | — | legs |
| Stewart top plate (carbon fiber, ~400 mm Ø) | — | platform surface |

— — — — —

## Quick start (WSL2 laptop, one double-click)

If USB-CAN and both IMUs are already usbipd attach-ed, the fastest

path from desktop to running GUI is the Windows launcher:

Double-click `scripts/start_stable_bot.bat` — brings up can0, starts

gui_server.py on :8080, runs ros2 launch, opens the browser. Two

WSL windows stay open so you can watch logs; close them to shut down.

Companion launchers in the same folder:

`scripts/stop_stable_bot.bat` — clean teardown (kills every Stable-Bot

process)

`scripts/reset_stable_bot.bat` — one-click hard reset + relaunch +

verify (wraps reset_stewart_stack.py --launch --verify)

### Manual start (three terminals)

If you prefer terminals or need finer control, three WSL panes sourced

to ROS 2 Kilted + workspace overlay:

source /opt/ros/kilted/setup.bash
source ~/ros2_ws/install/local_setup.bash

Terminal 1 — CAN bus up (needs sudo once per session):

sudo ip link set can0 up type can bitrate 1000000
sudo ip link set can0 txqueuelen 1000

Terminal 2 — HTTP server serving the GUI + recovery endpoints:

python3 ~/ros2_ws/src/stewart_bringup/scripts/gui_server.py

Terminal 3 — The ROS 2 stack:

ros2 launch stewart_bringup stewart_gui_launch.py

Open http://localhost:8080/ in a browser.

— — — — —

## Quick start (Pi — headless, auto-boot)

See [docs/PI_MIGRATION.md](docs/PI_MIGRATION.md) — includes headless

WiFi setup (phone hotspot), ROS 2 install, systemd services, and how to

control the robot from your laptop browser over WiFi.

— — — — —

## Typical session flow

Open GUI. Wait for the Status panel to populate (≤ 2 s).

Click **Read now** in the ODrive errors panel. All 6 should read `OK`.

If any don't, click Clear all; if they re-latch, power-cycle.

Run the homing section's **Stall-home this leg (bottom)** once per

leg (or all 6 selected together). Respond y in the stdin box when

prompted.

If you need a leveled reference, use the **Stabilization** card:

Arm → Level ON → drag Z slider to desired height → watch the

roll/pitch errors settle to green (<0.1°).

For rolling-ball or routine demos: record a routine with the **rec**

checkbox, then analyze it offline with

analyze_routine_log.py --all.

— — — — —

## One-command recovery

When anything goes wrong (GUI stuck on "waiting", ODrives red-pulsing,

CAN bus-off, etc.):

python3 ~/ros2_ws/src/stewart_bringup/scripts/reset_stewart_stack.py --launch --verify

This kills everything, frees port 9090, restarts the ROS daemon,

relaunches the bundle, and polls /status to confirm it actually came

up. Full diagnostics in the terminal; full log saved to

logs/last_launch.log.

For deeper trouble, see [docs/TROUBLESHOOTING.md](docs/TROUBLESHOOTING.md).

— — — — —

## Data output

Recorded routine runs land in ~/ros2_ws/src/stewart_bringup/logs/<timestamp>_<routine>/:

`routine.json` — exact copy of the routine played

`metadata.json` — motor limits, level cal, PI gains, feature flags

`telemetry.csv` — 50 Hz: commanded pose, IMU RPY/accel/gyro, encoder

positions, Iq, level corrections, per-leg modes

`can.log` — raw `candump -L` capture

`end.json` — final duration + timestamp

Bulk-compare all logs:

python3 ~/ros2_ws/src/stewart_bringup/scripts/analyze_routine_log.py --all

Writes logs/summary.csv with one row per run + ball-trajectory metrics.

— — — — —

## Further reading

[`docs/ARCHITECTURE.md`](docs/ARCHITECTURE.md) — process graph, topic flow, control algorithms

[`docs/PI_MIGRATION.md`](docs/PI_MIGRATION.md) — Phase 8: move backend to a Raspberry Pi

[`docs/TROUBLESHOOTING.md`](docs/TROUBLESHOOTING.md) — consolidated fixes for all the issues seen during bring-up