# Stable-Bot Architecture

## Process graph

┌──────────────────────────┐
     HARDWARE                  PROCESSES       │   BROWSER (on any host)  │
  ──────────────            ─────────────────   │                          │
                                               │   web GUI (index.html)   │
                                               │   roslibjs + fetch()     │
                                               └────────┬─────────────────┘
                                                        │ ws: 9090   http: 8080
                                               ┌────────▼─────────┐
                                               │ rosbridge (9090) │
                                               │ gui_server (8080)│
                                               └────────┬─────────┘
                                                        │ ROS topics
                  ┌──────────────┐              ┌───────▼──────────────┐
  MTi-630 #1 ────►│ xsens node   │─/base/*─────►│                      │
  MTi-630 #2 ────►│ xsens node   │─/platform/*─►│                      │
                  └──────────────┘              │                      │
                                                │  stewart_control     │
  USB-CAN ─── can0 ────────┐                    │       _node          │
                           │                    │                      │
   ┌── ODrive 0 ──┐         │                    │  (rclpy executor +   │
   ┌── ODrive 1 ──┤         │                    │   background threads)│
   ┌── ODrive 2 ──┤ <─ CAN ─┘                    │                      │
   ┌── ODrive 3 ──┤         │                    │                      │
   ┌── ODrive 4 ──┤         │                    │                      │
   ┌── ODrive 5 ──┘         │                    └──────────────────────┘
                            │                           ▲
                            └──── ODriveFeeder ─────────┤
                            └──── EncoderListener ──────┤
                                                        │
                            (optional) stall_home subproc when homing

## Threads inside `stewart_control_node`

| Thread | Rate | Responsibility |
| --- | --- | --- |
| rclpy executor | event-driven | service calls, topic callbacks, timers |
| ODriveFeeder | 50 Hz | per-leg `Set_Input_Pos` (with vel_ff) or `Set_Input_Vel` |
| EncoderListener RX | blocking `bus.recv` | absorb `Get_Encoder_Estimates` (0x009), `Get_Error` (0x003), `Get_Iq` (0x014) |
| EncoderListener TX | 10 Hz / 2 Hz | RTR request encoders + errors/Iq so cyclic broadcast isn't required |
| Level loop | 50 Hz | PI control with dynamic reference + velocity feedforward integration |
| Routine player | 100 Hz | linear interpolation between keyframes, forwards to `_do_set_pose` |
| Recording sampler | 50 Hz | append a row to `telemetry.csv` during `rec`-enabled playback |
| Homing pump | blocking | stream stall_home subprocess stdout to `/homing_output` |
| CAN reset worker | on-demand | background `ip link down/up` without blocking rclpy |

## Topics

| Topic | Type | Publisher | Rate | Purpose |
| --- | --- | --- | --- | --- |
| `/status` | `std_msgs/String` (JSON) | control_node | 2 Hz | armed flags, IMU freshness, level errors, routine state |
| `/leg_encoders` | `Float64MultiArray` (6) | control_node | 20 Hz | motor positions in turns |
| `/leg_currents` | `Float64MultiArray` (6) | control_node | 20 Hz | Iq_measured in amps |
| `/platform_rpy` | `Float32MultiArray` (3) | control_node | 20 Hz | IMU roll/pitch/yaw |
| `/odrive_errors` | `std_msgs/String` (JSON) | control_node | 2 Hz | per-node active_errors + disarm_reason |
| `/platform/imu/data` | `sensor_msgs/Imu` | xsens (platform) | 400 Hz | main leveling feedback |
| `/base/imu/data` | `sensor_msgs/Imu` | xsens (base) | 400 Hz | not yet used (future: wave rejection) |
| `/control_cmd` | `std_msgs/String` (JSON) | browser | on-demand | **primary action bus** (see below) |
| `/control_result` | `std_msgs/String` (JSON) | control_node | after each cmd | replies to `/control_cmd` |
| `/jog_vel_cmd` | `std_msgs/Float32` | browser | ~10 Hz while dragging | dead-man velocity slider |
| `/homing_output` | `std_msgs/String` | control_node | per line | stall_home subprocess stdout |
| `/homing_stdin_in` | `std_msgs/String` | browser | on Enter | stdin piped to stall_home |

## `/control_cmd` dispatcher

Rather than rely on ROS services (which have an intermittent discovery

bug in rosbridge on WSL2), every GUI action is a JSON message on

/control_cmd:

{"cmd": "jog_leg", "leg": 0, "delta_turns": 0.1}
{"cmd": "set_pose", "x": 0, "y": 0, "z": 50, "roll": 0, "pitch": 0, "yaw": 0}
{"cmd": "play_routine", "name": "demo_wave", "loop": false, "record": true}

Every command is dispatched to the same internal methods the

equivalent ROS service handler uses, so both paths behave identically.

Replies land on /control_result with the same cmd field plus

success and message. Supported cmd values include:

arm / disarm: `activate`, `deactivate`, `e_stop`, `arm_leg`

motion: `jog_leg`, `set_pose`, `go_to_rest`, `enable_level`

limits: `set_speed_cap`, `set_leg_current`

errors: `read_errors`, `clear_errors`

homing: `start_homing`, `cancel_homing`

routines: `list_routines`, `start_recording`, `record_keyframe`,

clear_recording, save_recording, play_routine, stop_routine,

delete_routine, generate_rolling_ball_routine

diagnostics: `probe_graph`, `reset_can`, `restart_ros2_daemon`

## Recovery path (when things break)

`scripts/reset_stewart_stack.py --launch --verify` — the go-to.

GUI Diagnostics panel: **View launch log**, **Hard reset stack**,

(Re)launch stack, Reconnect rosbridge, Restart ros2 daemon

— all bypass rosbridge via the gui_server.py HTTP endpoints.

`scripts/read_odrive_errors.py --clear` — direct CAN to clear ODrive

error latches.

USB cycle the CAN adapter via `usbipd detach/attach` (WSL2) or

physical replug (Pi).

## Key algorithmic choices

**Per-leg mode feeder** — each leg independently in `pos`, `vel`, or

idle. Velocity-mode legs receive Set_Input_Vel(shared_vel);

position-mode legs receive Set_Input_Pos(target, vel_ff=dpos/dt);

idle legs get nothing. Avoids fighting between pose-hold and dead-man

jog.

**Velocity feed-forward from finite-diff target** — collapses dynamic

tracking error. Required when the PI gain is too low to track a

moving reference (see the rolling-ball session notes).

**Dynamic level reference** — PI drives `IMU_rpy → level_ref +

current_rpy rather than a static level_ref`. Means commanded tilt is

actually achieved in the world frame; routine commands aren't

cancelled by the PI.

**Watchdog strategy** — feeder runs at 50 Hz *before* any axis enters

CLOSED_LOOP and continues until after every axis reaches IDLE. This

prevents the 500 ms ODrive watchdog from ever firing mid-transition.

(See feedback_odrive_watchdog.md memory.)

**Homing releases the bus** — stall_home.py runs as a subprocess with

python -u; the node closes its CAN socket first, then reacquires

after the subprocess exits. Stdout is pumped byte-by-byte with a

150 ms idle flush so input() prompts show up in the GUI log

without waiting for the reply.