# Phase 8 — Raspberry Pi Migration

Goal: move the backend (CAN + IMUs + control node + rosbridge + GUI

server) off the laptop and onto a Raspberry Pi mounted on/near the

Stewart platform. Laptop becomes a thin client — a browser pointed at

the Pi's GUI over WiFi. No more usbipd, no more "unplug and replug the

CAN adapter," no more WSL2 DDS quirks.

## Hardware list

**Raspberry Pi 4B 4GB+ or Pi 5** (Pi 5 preferred — ROS 2 Kilted is

compile-heavy on armhf, and the Xsens driver benefits from the extra

cores)

32GB+ A1/A2 microSD card

5V USB-C PSU (3A min for Pi 4, 5A recommended for Pi 5)

**Your existing** USB-CAN gs_usb adapter, 2× Xsens MTi-630, 6× ODrive Pro

Laptop with a browser (connects to the Pi's IP)

Optional: a 4-port powered USB hub so the Pi can feed the IMUs and

CAN adapter together

## One-time SD card setup (answers your WiFi-hotspot question)

Use Raspberry Pi Imager from any OS. It has a pre-boot configuration

panel that handles headless WiFi, SSH, and hostname in one go — this

is the simplest way to make a Pi auto-connect to a phone hotspot

without attaching a screen.

Download **Raspberry Pi Imager** (Windows/Mac/Linux).

Choose OS → **Other general-purpose OS → Ubuntu → Ubuntu Server

24.04 LTS (64-bit)**.

*(Required for ROS 2 Kilted. Raspberry Pi OS doesn't currently have

prebuilt Kilted binaries; Ubuntu 24.04 does.)*

Choose storage → your microSD card.

**Click the gear icon** (or `Ctrl+Shift+X`) → this opens the

Advanced Options panel:

✅ **Set hostname**: `stablebot` (so you can reach it as

stablebot.local via mDNS)

✅ **Enable SSH** → "Use password authentication" (or keys — your

choice)

✅ **Set username and password**: `sorak` / (your pick)

✅ **Configure wireless LAN**:

**SSID**: name of your phone's hotspot (case-sensitive; must be

exact)

**Password**: your hotspot password

**Wireless LAN country**: set to your country (US, GB, DE, etc.)

— required or WiFi won't come up

**Set locale settings** (time zone + keyboard layout)

Save → Write. The Imager bakes these into `system-boot/` and

writable/ on the SD card so the Pi configures itself on first boot.

Eject, insert into Pi, plug in power.

**Turn on your phone's hotspot** (same SSID + password you entered).

Wait ~60 s. Check your phone's hotspot "connected devices" list —

you should see the Pi appear. Note its IP (e.g. 192.168.43.42).

### Connecting multiple networks (phone hotspot + home/lab WiFi)

If you want the Pi to prefer your lab WiFi when available and fall

back to the phone hotspot, after first boot edit

/etc/netplan/50-cloud-init.yaml (Ubuntu 24.04 uses netplan):

network:
  version: 2
  wifis:
    wlan0:
      dhcp4: true
      access-points:
        "LabWiFi":
          password: "labpass"
        "PhoneHotspot":
          password: "phonepass"

Then sudo netplan apply.

Ubuntu will try access points in the order listed, rotating if the

first isn't in range. For priority weighting, use NetworkManager's

nmcli instead.

### Finding the Pi without a screen

**mDNS** (simplest, usually works): `ping stablebot.local` from your

laptop on the same network. If it resolves, SSH in with

ssh sorak@stablebot.local.

**Phone hotspot clients list**: shows every device + IP.

**nmap** of your local subnet: `nmap -sn 192.168.43.0/24` (adjust

for your hotspot range).

**Router admin page**: if on lab WiFi, the router usually lists DHCP

leases.

## First boot on the Pi

Once SSH'd in:

ssh sorak@stablebot.local

# Update and install system deps
sudo apt update && sudo apt upgrade -y
sudo apt install -y curl gnupg lsb-release git python3-pip python3-venv \
                     can-utils build-essential

# Setup the ROS 2 Kilted apt repo
sudo curl -sSL https://raw.githubusercontent.com/ros/rosdistro/master/ros.key \
    -o /usr/share/keyrings/ros-archive-keyring.gpg
echo "deb [arch=$(dpkg --print-architecture) signed-by=/usr/share/keyrings/ros-archive-keyring.gpg] \
http://packages.ros.org/ros2/ubuntu $(. /etc/os-release && echo $UBUNTU_CODENAME) main" \
    | sudo tee /etc/apt/sources.list.d/ros2.list > /dev/null

# Install ROS 2 base + rosbridge
sudo apt update
sudo apt install -y ros-kilted-ros-base ros-kilted-rosbridge-suite \
                     ros-kilted-rclpy python3-colcon-common-extensions \
                     python3-rosdep
sudo rosdep init
rosdep update

# Python deps
pip3 install --break-system-packages python-can pyyaml numpy

## Clone + build the workspace

mkdir -p ~/ros2_ws/src
cd ~/ros2_ws/src
# Copy or git-clone:
git clone https://github.com/YOUR-REPO/stewart_bringup.git
git clone https://github.com/YOUR-REPO/jugglebot_interfaces.git
# Xsens driver (required — not in apt):
git clone https://github.com/xsens/xsens_mti_ros2_driver.git

cd ~/ros2_ws
rosdep install --from-paths src --ignore-src -r -y
source /opt/ros/kilted/setup.bash
colcon build --symlink-install
echo "source ~/ros2_ws/install/local_setup.bash" >> ~/.bashrc

## udev rules for stable device names

The gs_usb CAN adapter and Xsens IMUs present at /dev/ttyUSB0,

/dev/ttyUSB1, etc. — but the numbering isn't deterministic. Use udev

rules to pin them to stable paths. The existing laptop setup has these

at /dev/imu_mti630 and /dev/imu_mti630_b — replicate on the Pi:

sudo tee /etc/udev/rules.d/60-stablebot.rules <<'EOF'
# Xsens MTi-630 — two units, pin by serial number
SUBSYSTEM=="tty", ATTRS{idVendor}=="2639", ATTRS{serial}=="DBBBRHGX", \
    SYMLINK+="imu_mti630",   MODE="0666"
SUBSYSTEM=="tty", ATTRS{idVendor}=="2639", ATTRS{serial}=="DBBBRHBS", \
    SYMLINK+="imu_mti630_b", MODE="0666"
# gs_usb CAN adapter permissions (no symlink — kernel names it can0)
SUBSYSTEM=="net", ATTRS{idVendor}=="1d50", ATTRS{idProduct}=="606f", \
    GROUP="dialout", MODE="0666"
EOF
sudo udevadm control --reload-rules && sudo udevadm trigger
sudo usermod -aG dialout sorak
# log out + back in for group change to take effect

## Bring up can0 on boot

Add a systemd network unit (or /etc/network/interfaces.d entry, or

netplan snippet — Ubuntu 24.04 prefers netplan/networkd). Simplest:

sudo tee /etc/systemd/network/80-can0.network <<EOF
[Match]
Name=can0

[CAN]
BitRate=1000000
EOF
sudo networkctl reload

This brings can0 up at 1 Mbps every boot automatically.

## Auto-start services on boot

Two systemd units — one for the ROS 2 launch, one for the GUI HTTP server:

sudo cp ~/ros2_ws/src/stewart_bringup/scripts/stable_bot.service \
        /etc/systemd/system/
sudo cp ~/ros2_ws/src/stewart_bringup/scripts/stable_bot_gui.service \
        /etc/systemd/system/
sudo systemctl daemon-reload
sudo systemctl enable --now stable_bot_gui.service stable_bot.service

Check status any time:

systemctl status stable_bot.service stable_bot_gui.service
journalctl -u stable_bot.service -f        # live logs

## Access from the laptop

Point your laptop's browser at `http://stablebot.local:8080/`

(or http://<pi-ip>:8080/ if mDNS is flaky). That's it — the browser

talks to the Pi's rosbridge at ws://stablebot.local:9090 and its

GUI server at :8080. Zero ROS setup on the laptop.

If connecting from a different subnet, you may need to adjust ROS DDS

discovery — see the "multiple machines" section of the ROS 2 docs.

Within a single WiFi network it just works with default discovery.

## Power-cycle recovery

The gs_usb adapter occasionally needs a physical replug when bus-off

happens with flaky cabling. On the Pi (no usbipd required):

Physically unplug USB-CAN, wait 3 s, replug → can0 re-appears via the

systemd-networkd unit above.

`sudo systemctl restart stable_bot.service` to restart the ROS stack

— or just use the Hard reset stack + (Re)launch stack buttons

in the web GUI, same as the WSL2 workflow.

## What does NOT change from the WSL2 setup

GUI (`index.html`) — same file served by `gui_server.py`.

Launch file, control node, all scripts — same code.

Recording format and analyzer — same.

`reset_stewart_stack.py` still works for hard resets.

The Pi migration is really just: "run all the same stuff, but on a

different Linux box that happens to have native USB and no WSL2

weirdness."

## Laptop-only mode after migration

You can still run the WSL2 stack alongside the Pi — they don't conflict

because each has its own CAN bus (WSL2 via usbipd, Pi via native USB).

Useful for development: iterate code on the WSL2 side, rsync to the Pi

when happy:

rsync -av --delete \
  ~/ros2_ws/src/stewart_bringup/ \
  sorak@stablebot.local:/home/sorak/ros2_ws/src/stewart_bringup/
ssh sorak@stablebot.local \
  "cd ~/ros2_ws && colcon build --packages-select stewart_bringup --symlink-install && \
   sudo systemctl restart stable_bot.service"