# Troubleshooting

Consolidated fixes for issues encountered during bring-up. Organized

from most-common to least-common. The one-command fix at the top is the

right answer about 70% of the time.

— — — — —

## The one command

python3 ~/ros2_ws/src/stewart_bringup/scripts/reset_stewart_stack.py --launch --verify

Kills everything, frees port 9090, restarts the ROS daemon, relaunches

the bundle, polls /status for 20 s, and prints a verdict. If it says

/status IS publishing, you're good; close and reopen the browser tab.

— — — — —

## Symptom → fix

### GUI shows "(connected but no /status message received yet…)"

Run the one command above.

If verify still fails, check the log tail it prints. Usually one of:

**"Address already in use" on 9090** → a zombie rosbridge. The

script's step 0 (fuser -k 9090/tcp) fixes this. Re-run.

**No `stewart_control_node ready` line** → the control node is

crashing during init. Run it standalone to see the traceback:

ros2 run stewart_bringup stewart_control_node.

**Node is ready but CLI can't see topics** → DDS env mismatch.

Make sure the launch file does NOT set ROS_LOCALHOST_ONLY or

ROS_AUTOMATIC_DISCOVERY_RANGE. (See memory

feedback_ros2_discovery_env.md.)

### ODrives pulsing red

Latched error state. Open the GUI errors panel, click Read now to

see which flag:

| Flag | Meaning | Fix |
| --- | --- | --- |
| `WATCHDOG_TIMER_EXPIRED` | host stopped sending commands | Click **Clear all**. Usually caused by killing the node while armed; the feeder-before-arm pattern prevents it. |
| `CURRENT_LIMIT_VIOLATION` | motor wanted more current than cap | Raise current slider in Motor Limits panel. |
| `SPINOUT_DETECTED` | motor stalled / fought mechanical bind | Clear errors, reduce load or current, investigate mechanics. |
| `DC_BUS_OVER_CURRENT` | PSU can't sustain demand | Lower current limit, check PSU sizing. |

If Clear all doesn't work (drives don't respond), the CAN bus is

off. Click Soft-reset CAN bus — if that times out,

physically unplug + replug the USB-CAN cable.

### CAN "Transmit buffer full"

The kernel TX queue is saturated because sends are failing (ODrives in

error state aren't ACKing). Cycle:

sudo ip link set can0 down
sudo ip link set can0 up type can bitrate 1000000

If that returns Timer expired, the gs_usb adapter itself is wedged —

physically unplug USB, wait 3 s, replug, and re-usbipd attach if on

WSL2.

### `ros2 node list` hangs

ROS 2 daemon is stale. Either:

GUI Diagnostics → **Restart ros2 daemon**, or

Terminal: `ros2 daemon stop && ros2 daemon start`.

### Xsens IMU "No MTi device found"

USB not attached. On WSL2: usbipd attach --wsl --busid <X-Y> from

Windows admin PowerShell. On Pi: verify the udev symlinks exist:

ls /dev/imu_mti630*. If they don't, check dmesg | tail for USB

enumeration errors.

### Rolling ball rolls right off the disk

Open-loop + static friction is fundamentally hard. In order of

effectiveness:

**Physically flick the ball** before pressing Play so it's moving

when the routine starts.

Swap to a **smoother surface** (acrylic, glass, polished aluminum,

or a sheet of paper taped to the carbon fiber).

Use a **heavier / more-dense ball** (glass marble, ball bearing).

Tune the routine: smaller `radius_m`, `max_tilt_deg`, try the

tilt_multiplier slider.

**Give up and use the stabilization demo** — hold level, tilt the

base by hand, top stays flat. Same platform, more impressive

showcase, works every time.

### Browser shows "Can 'Upgrade' only to 'WebSocket'"

You've navigated to http://localhost:9090 (the rosbridge port).

Navigate to http://localhost:8080/ instead (the GUI server port).

### Everything seems fine but no motor movement

Drives armed but at the wrong mode. Check:

GUI **per-leg arming** row: each checkbox should be ticked (green)

for velocity mode, or use the main Arm button for position mode.

Motor limits slider isn't at minimum.

For per-leg jog (+ / − buttons), the leg auto-switches to pos mode.

If it doesn't respond, check the errors panel — might be latched.

### GUI buttons do nothing but /status is flowing

/control_cmd messages aren't reaching the control node. Usually a

rosbridge reconnect fixes it:

GUI Diagnostics → **Reconnect rosbridge**.

Or: close the browser tab completely, reopen to

http://localhost:8080/.

### Pi: SSH refused / `stablebot.local` doesn't resolve

Phone hotspot must be ON when the Pi boots, with the SAME SSID and

password you baked into the SD card. Case-sensitive.

Give it ~60 s after power-on.

mDNS (`.local`) may be blocked on phone hotspots — check the

hotspot's "connected devices" list and SSH to the raw IP instead.

### Pi: launch comes up but IMUs fail with "No MTi device"

udev rules don't know your serial numbers yet. Check:

udevadm info -a /dev/ttyUSB0 | grep serial

Copy the serial(s) into /etc/udev/rules.d/60-stablebot.rules,

sudo udevadm control --reload-rules && sudo udevadm trigger.

— — — — —

## Recovery hierarchy (when to escalate)

Close browser tab completely, reopen → fixes ~30% of GUI hangs

**Reconnect rosbridge** button → fixes websocket wedges

**Restart ros2 daemon** button → fixes CLI hangs

**Hard reset stack + (Re)launch stack** buttons → fixes zombie processes

`reset_stewart_stack.py --launch --verify` in terminal → the big hammer

`reset_stewart_stack.py --nuclear --launch --verify` → cleans stale python3 procs

Physical USB cycle + `usbipd attach` (WSL2) or replug (Pi) → last resort

Power-cycle the ODrive supply → ODrives won't clear via CAN

If any step before a later one is usable, skip to it. Earlier steps are

less disruptive.