# Level-loop PI Tuning Plan

Date drafted: 2026-04-26 Owner: Luke Status: ready to start (DEFER until ODrive recommutation is done)

## Why

The level loop in stewart_control_node._level_run (KP=0.7, KI=0.2, deadband=0.05°, rate-limit=0.2°/iter, MAX_CORR=5°, alpha=0.3) does not settle inside the 0.1° error band. Symptom is a slow limit cycle around the target, not high-frequency jitter, so IMU noise (sub-millidegree on the MTi-630) is ruled out. Hand-tuning so far has not converged. We want a data-driven tuning loop instead.

## Pre-condition (do this first)

Re-do ODrive motor + encoder offset calibration on every node. Do not tune gains against a plant whose commutation is broken — gains fit to a broken plant will be wrong as soon as the plant is fixed.

## Phase 1 — Instrument the loop (control_node)

Publish a single dense /level_diag topic at 50 Hz with the full controller state per tick. Don’t try to recover this from existing topics (sampled at different rates, can’t align integrator state).

Fields (all in one custom msg):

- t_imu (IMU receive monotonic), t_tick (tick start)

- roll, pitch (raw IMU)

- target_roll, target_pitch (commanded — current_rpy + level_ref)

- err_r, err_p, err_r_filt, err_p_filt

- integ_r, integ_p

- pi_out_r, pi_out_p (= -err*KP + integ — unclipped)

- corr_r, corr_p (= tilt_*_corr after rate-limit + MAX_CORR clamp)

- clip_flags (bitfield: rate-limited / max-corr clipped / deadband)

- motor_targets[6], leg_enc[6], leg_iq[6]

- dt_actual (loop jitter — important if >> 20 ms)

Use a custom msg in jugglebot_interfaces (e.g. LevelDiag.msg) to avoid Float32MultiArray-with-magic-indices.

## Phase 2 — Bag recording in the GUI

Two ROS services on stewart_control_node:

- /level_record/start (string name, optional) → spawns ros2 bag record -o <path> /level_diag /platform_rpy /leg_encoders /leg_currents /control_cmd Default path ~/stable_bot_bags/<UTC-timestamp>_<name>.

- /level_record/stop → SIGINT to the subprocess; returns final path + duration + size.

GUI additions (un-hide the Active Stabilization panel for this work):

- “Record” / “Stop” toggle button + filename input + size/duration readout

- “Step test” button: programmatic disturbance — commands roll = +1° for 3 s, then back to 0. Repeatable so step responses are comparable across runs.

- “Recent bags” list with download/delete (served by gui_server.py).

## Phase 3 — Offline analysis script

scripts/analyze_level_bag.py using rosbag2_py + matplotlib + numpy:

Plots:

- err_filt vs time

- integ vs time

- corr vs time

- FFT of err_filt to find the limit-cycle frequency

Step metrics:

- Rise time, peak overshoot, 5 %-settling time, 1 %-settling time

Steady-state stats:

- Mean, std-dev, peak-to-peak of err_filt over the last 5 s

Saturation count:

- % of ticks where clip_flags != 0. High % = controller is fighting the rate-limiter, gains are too aggressive.

## Phase 4 — Tuning protocol

- Baseline bag (after recommutation): 30 s idle, level_hold engaged, no disturbance. Measure RMS error and dominant frequency from the FFT.

- Step bag: 5 step tests (+1° roll / 3 s hold / back to 0). Average the responses to reduce noise.

- From the step response:

  - Overshoot > 10 % and oscillates: KP too high, or KP/KI ratio wrong. Try KI = KP/3 instead of current KP/3.5.

  - Sluggish with steady-state error: KI too low; raise toward KP/2.

  - High-freq oscillation (> 3 Hz): rate-limit can’t catch the controller’s bandwidth — lower KP first.

  - Iterate, re-record, compare. Stop when settling-time(0.1°) ≤ 0.5 s and steady-state RMS ≤ 0.05°.

- Lock gains in config/level_gains.yaml rather than as Python constants. Tuning script can then sweep them via a parameter file without code changes.

## Build order

- Phase 1 + 2 together → push → deploy

- Phase 3 script

- Phase 4 (iterative, user-driven; I just maintain the plotting script)

## Rough size

- Phase 1: ~80 lines (control_node + msg)

- Phase 2: ~150 lines (services + GUI panel + gui_server endpoints)

- Phase 3: ~250 lines (analysis script)

- Custom msg requires rebuilding jugglebot_interfaces (one extra colcon step on the Pi).